RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY/AUGUST 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electric Drives | Max. Marks | : | 100 |
| Subject Code | : | EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit - Make necessary assumptions wherever necessary.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Using four quadrant diagrams, analyze the multi-quadrant operation of a electric bus drive system. State the conventions used and assumptions made. | CO1 | (10) |
|  | b) | With the help of a block diagram, describe the structure of an electric drive system. | CO1 | (07) |
|  | c) | List any four advantages of electric drives. | CO1 | (03) |
| 2. | a) | Write short notes on the different component of load torque and sketch its speed-torque characteristics. | CO1 | (08) |
|  | b) | A motor drives two loads. One has rotational motion. It is coupled to the motor through a reduction gear with gear ratio of 0.1 and efficiency of 90%. The load has a moment of inertia of 10kg-m2 and torque of 10N-m. Other load has translational motion and consists of 1000kg weight to be lifted up at an uniform speed of 1.5m/s. Coupling between this load and the motor has an efficiency of85%. Motor has an inertia of 0.2kg-m2 and runs at a constant speed of 1420 rpm. Determine the equivalent inertia referred to the motor shaft and power developed by the motor. | CO1 | (06) |
|  | c) | Write short notes on choices of electrical drives. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Which kind of rectifier would you suggest to operate a dc motor in forward motoring and reverse braking quadrant? Justify your suggestion. | CO2 | (07) |
|  | b) | A 220V, 970 rpm, 100A DC separately excited motor has an armature resistance of 0.05Ω. It is braked by plugging from an initial speed of 1000rpm. Calculate: - Resistance to be placed in armature circuit to limit braking current to twice the full-load value. - Braking torque. - Torque when the speed has fallen to zero. | CO2 | (08) |
|  | c) | Briefly explain about the armature voltage control method of a dc motor. | CO2 | (05) |
| 4. | a) | A dc motor needs to be operated in 1st and 4th quadrant. If the available source is dc in nature, what topology of converter will you suggest to meet the above mentioned requirement? Explain the working of this converter with relevant waveforms. | CO2 | (08) |
|  | b) | A 200V, 875rpm, 150A separately excited dc motor has an armature resistance of 0.06Ω. It is fed from a single phase fully-controlled rectifier with an AC source voltage of 220V, 50Hz. Assuming continuous conduction, calculate: - Firing angle for rated motor torque and 750 rpm - Motor speed for α=160° and rated torque. | CO2 | (08) |
|  | c) | When varying speed by flux control, flux must be varied in small steps only. Why? | CO2 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain the regenerative braking of an induction motor with diagrams. | CO3 | (07) |
|  | b) | A star connected squirrel cage induction motor has the following ratings and parameters: 400V, 50Hz, 4-pole, 1370rpm, Rs=2ohm, Rr’=3ohm, Xs=Xr’=3.5ohm. Motor is controlled by a voltage source inverter. - Speed for a frequency of 30Hz and 80% of full-load torque - Frequency for a speed of 1000rpm and full-load torque - Torque for a frequency of 40Hz and speed of 1100rpm. | CO3 | (07) |
|  | c) | With a neat diagram, explain the operation of star-delta starter used for starting three phase induction motor. | CO3 | (06) |
| 6. | a) | With regard to induction motors, explain the dc dynamic braking method. | CO3 | (06) |
|  | b) | A 400V, star connected, 3-phase, 6-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1 ohm, Xs=Xr’=2 ohm. This motor is to be braked by plugging from its initial full load speed of 950rpm. Stator to rotor turns ratio is 2:3. - Calculate the initial braking current and torque as a ratio of their full load values - What resistance must be inserted in rotor circuit to reduce the maximum braking current to 1.5 times full load current? What will be initial braking torque now? | CO3 | (06) |
|  | c) | If the maximum torque capability is to be retained at low speeds, which type of speed control method should be used? Elaborate your answer with reasons. | CO 3 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Discuss about different kinds of starting methods of a synchronous motor. | CO4 | (10) |
|  | b) | Explain Hall effect. | CO4 | (03) |
|  | c) | Draw the circuit diagram of a voltage source inverter fed BLDC motor drive and explain its operation. | CO4 | (07) |
| 8. | a) | Derive an expression for the torque produced by a cylindrical rotor wound field motor and draw the speed-torque characteristics. | CO4 | (07) |
|  | b) | With diagrams, describe the constructional feature of different types of rotors in BLDC motors. | CO4 | (07) |
|  | c) | Draw the speed-torque characteristics of a BLDC motor and write short notes on it. | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the operation of unipolar drive circuit used in stepper motor control using circuit diagram. | CO5 | (07) |
|  | b) | A 4-phase, 4/2-pole variable reluctance stepper motor is to be controlled for half-step operation. Find out the switching sequence and draw the switching table, to rotate the rotor continuously. Also find the step-angle that can be obtained in half-step operation. | CO5 | (05) |
|  | c) | Discuss about the different classes of motor duty with relevant load waveforms. | CO5 | (08) |
| 10. | a) | A motor operates on a periodic duty cycle in which it is clutched to its load for 10minutes and declutched to run on no-load for 20minutes. Minimum temperature rise is 40°C. Heating and cooling time constants are equal and have a value of 60 minutes. When load is declutched continuously the temperature rise is 15°C. Determine, - Maximum temperature during the duty cycle - Temperature when the load is clutched continuously. | CO5 | (06) |
|  | b) | Explain the principle of operation of a stepper motor with necessary diagrams. | CO5 | (08) |
|  | c) | Write short notes on the different operating regions of stepper motor’s speed torque characteristics. | CO5 | (06) |

****************