RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE51 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With the help of block diagram describe digital signal processing system. | CO1 | (06) |
|  | b) | Compute the circular convolution of the following sequences using DFT-IDFT method and compare the results using matrix method. x(n)= {2,1,2,1}, h(n)= {1,2,3,4}. | CO1 | (10) |
|  | c) | Compare linear and circular convolution. | CO3 | (04) |
| 2. | a) | Determine the response of the FIR filter whose impulse response is given as h(n)={1,2}when an input x(n)={2,1,3} is applied. Use circular convolution and verify the result using linear convolution. | CO3 | (08) |
|  | b) | Using DFT-IDFT, determine the response of the FIR filter with impulse response h(n)= {2,3,1,1} to the input sequence x(n)= {1,3,5,3}. | CO1 | (08) |
|  | c) | Explain the advantages of DFT in digital signal processing. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | The sequence x(n)=; 0n is filtered through a filter impulse response h(n)={1,-1}.compute the output of the filter using overlap add method. Use 5-point circular convolution. | CO1 | (10) |
|  | b) | Develop DIF FFT Radix 2 algorithm to compute the DFT X(k) of signal x(n). | CO1 | (10) |
| 4. | a) | The sequence x(n)={1,1,0,3,2,1,9,7,-3,9,6,-1,2,0,2,1} is filtered through a filter whose impulse response h(n)={3,2,1,1,}.compute the output y(n) using overlap and save method. Use9-point circular convolution. | CO1 | (10) |
|  | b) | The first five points of an 8-point DFT of the real-valued sequences are: X(0) = 0.5, X(1) = -j0.3, X(2) = 0=X(3), X(4) = 0.25 – j0.05. Determine the DFT of the remaining. Define the property involved. Also find the real point sequence using DIT FFT algorithm. | CO1 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain different structural realization of IIR filters. Obtain the cascade realization of . | CO4 | (12) |
|  | b) | A system is represented by the transfer function: H(z)=(2/3)z-2+1+ (2/3)z-1. Give a difference equation representation of this system and realize by linear phase structure. | CO4 | (08) |
| 6. | a) | Compare FIR and IIR filters. Realize a linear phase FIR filter with the following impulse response. Give necessary equations. h(n) = δ(n) + (1/2)δ(n-1) - (1/4)δ(n-2) + (1/2)δ(n-3) +δ(n-4). | CO4 | (12) |
|  | b) | Obtain parallel form realization of: | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | A filter is to be designed with the following desired frequency response: Hd(ω)= { 0, -/4 ≤ ω ≤ /4 { e-j2 ω , /4 ≤│ ω │ ≤ , Find the frequency response of the FIR filter designed using a Hamming window . | CO3 | (08) |
|  | b) | Design a 17-tap linear-phase FIR filter with a cutoff frequency wc = /2. The design is to be done based on frequency sampling technique. (consider low pass filter). | CO3 | (08) |
|  | c) | Explain Gibb’s phenomenon. | CO3 | (04) |
| 8. | a) | Obtain the coefficients of an FIR filter to meet the specifications given below using the window method 0.99 ≤│Hd(w)│ ≤ 1.01 , 0 ≤│w│ ≤ 0.375  │Hd(w)│ ≤ 0.01 , 0.5 ≤│w│ ≤ . | CO4 | (10) |
|  | b) | We would like to design an equiripple high pass filter of order N=64. The stopband ripple is to be no larger than δs =0.001 & the passband ripple no larger than δp =0.01. If we want a passband cut-off frequency equal to wp=0.72 , what will the stopband cut-off frequency be approximately equal to? | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Design a digital filter using Chebyshev approximation and Bilinear transformation to meet the following specifications: Pass band magnitude characteristics that is constant to within 1dB for frequencies 0≤ ω≤0.15  and stop band attenuation of at least 20dB for frequencies between ω=0.45  and . | CO4 | (15) |
|  | b) | Explain the architecture of C2xx DSP CPU. | CO5 | (05) |
| 10. | a) | Design the Chebyshev low pass filter using bilinear transformations with frequency response: 0.8 ≤ H(ω)≤1 for 0≤¦ω¦≤0.2, H(ω)≤0.2 for 0.32 ≤¦ω¦≤ | CO4 | (15) |
|  | b) | Explain briefly the design of IIR filters from analog filters. | CO4 | (05) |

****************