RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

MAKEUP EXAMINATIONS – MARCH 2017

| Course & Branch | : | Electrical & Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE501 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With the help of block diagram describe digital signal processing system. | CO1 | (06) |
|  | b) | By means of DFT-IDFT, determine the response of the FIR filter with impulse response h(n)= {1,2,3} to the input sequence x(n)= {1,2,2,1}. Also explain the aliasing problem due to the wrong selection of the size of DFT for the system given. | CO1 | (08) |
|  | c) | Determine 8 point DFT of x(n)={1,1,1,1,1} and sketch magnitude and phase spectrum. | CO3 | (06) |
| 2. | a) | Let x(n)={2,1,0,8}).Evaluate the 4 point DFT using matrix method.. | CO3 | (04) |
|  | b) | Given, g(n)= {2, 1, 0, 8} h(n)= {1, 9, 9, 5} Find the circular convolution using graphical method. Verify the result using multiplication method. | CO1 | (08) |
|  | c) | Consider the sequence: x (n)=δ(n)+2δ(n-2)+δ(n-3) (i) Find the four point DFT of x(n). (ii) If y(n) is, the four point circular convolution of x (n) with itself. Find y(n) and the four point DFT Y(k). | CO1 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | The sequence x(n)=cos 2πnN : 0≤n≤14;is filtered through a filter impulse response h(n)={1,-1}.compute the output of the filter using overlap add method. Use 5-point circular convolution. | CO1 | (10) |
|  | b) | Develop DIT FFT Radix 2 algorithm to compute the DFT X(k) of signal x(n). | CO1 | (10) |
| 4. | a) | The sequence x(n)={1,2,0,0,2,1,1,2,-3,5,6,-1,2,0,2,1} is filtered through a filter whose impulse response h(n)={3,2,1,1,}. Compute the output y(n) using overlap and save method. Use 9-point circular convolution. | CO1 | (10) |
|  | b) | The first five points of an 8-point DFT of the real-valued sequences are: X(0) = 0.25, X(1) = -j0.3018, X(2) = 0=X(3), X(4) = 0.125 – j0.0518. Determine the DFT of the remaining. Define the property involved and give the proof. Also find the real point sequence using DIF FFT algorithm. | CO1 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Design an ideal band pass filter with frequency response Hd(ejω)=1 for /4≤¦ω¦≤3π/4, using rectangular window with N=11. | CO4 | (10) |
|  | b) | Explain with example the design of linear phase FIR filter by frequency sampling method. | CO4 | (10) |
| 6. | a) | Write explanatory notes on : - Effects of window characteristics on filter response. - Advantages and disadvantages of FIR filter. | CO4 | (10) |
|  | b) | Design a FIR low pass filter with frequency response using rectangular window hd(ω)=e-jωc(N-1)/2 for -/2≤¦ω¦≤/2. = 0 elsewhere | CO4 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain the need of anti aliasing filters. Also explain different types of windows used in filter design. | CO4 | (10) |
|  | b) | Design a low pass digital filter to be used in an A/DH(z) D/A structure that will have a -3dB cutoff at 30 rad/sec and an attenuation of 50dB at 45 rad/sec. The filter is required to have a linear phase and the system will use a sampling rate of 100 samples/second. | CO4 | (10) |
| 8. | a) | A second order continuous time filter has the system function Ha(s)= 1S−a + 1s−b where a<0 and b<0 are real. (i) Determine the locations of the poles and zeroes of H(z) if the filter is designed using the bilinear transformation with Ts=2. | CO4 | (08) |
|  | b) | Design a digital low pass filter using Chebyshev filter design procedure that meets the following specifications: Pass band magnitude characteristics that is constant to within 1dB for frequencies below w=0.2π and stop band attenuation of atleast 15dB for frequencies between ω=0.3∏ and ∏. Use Bilinear transformation. | CO4 | (12) |
|  |  | UNIT- V |  |  |
| 9. | a) | An LTI system is described by the following input-output relation 2y (n) - y (n-2) – 4y (n-3) = 3x (n-2) Realize the system in the following forms: - Direct form I realization. - Direct form II realization. - Draw the transpose of the direct form II structure. | CO4 | (12) |
|  | b) | Realize a linear phase FIR filter with the following impulse response. Give necessary equations. | CO4 | (08) |
| 10. | a) | A discrete time IIR system H (z) is expressed as:- - Find the difference equation of the system. - Realize parallel form using second order sections. | CO4 | (12) |
|  | b) | Mention the application of lattice structure FIR filters. Given the coefficients km of the third order lattice filter corresponding to FIR filter as (0.1,0.2, 0.3). Find its impulse response and draw the equivalent direct form structure. | CO4 | (08) |

****************