RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS – JANUARY 2017

| Course & Branch | : | Electrical & Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE501 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT-I |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | a) | Discuss the advantages of Digital Signal Processing over Analog Signal Processing. |  |  | CO1 |  | (04) |  |
|  | b) | Compute i) DFT of unit sample δ(n) ii) 4 point DFT of the sequence cos(n∏/4) as linear transformation |  |  | CO3 |  | (06) |  |
|  | c) | Impulse response of a linear system is (1, -2, 2). Determine the output response of the system when the input is (1, 2, 1). Use circular convolution. Justify the selection of N. |  |  | CO2 |  | (10) |  |
| 2. | a) | Consider the sequence x(n)= u(n) - u(n-4), h(n) = u(n) - u(n-3). Evaluate circular convolution using DFT-IDFT method. Assume N=8. Plot the output. |  |  | CO2 |  | (08) |  |
|  | b) | Given two sequences x1(n) = (1,2,3,1), x2(n) = (4,3,2,2). Find x3 (n) such that X3(k) = X1(k) X2(k) using any two methods. |  |  | CO2 |  | (08) |  |
|  | c) | Write the circular time shift and circular frequency shift properties of DFT. |  |  | CO2 |  | (04) |  |
|  |  | UNIT-II |  |  |  |  |  |  |
| 3. | a) | The sequence x(n)= {1,2,3,4,4,2,3,1,1,2,4,4,1,1,5,7} is filtered through a filter whose impulse response is h(n)= {3, 2, 1, 1}. Compute the output of the filter y(n) using overlap save method. Use 9 point circular convolution, |  |  | CO2 |  | (10) |  |
|  | b) | Given, the sequence x(n)=sin((∏/2)n), 0 < n < 7, compute the 8-point DFT using DIFFFT algorithm. |  |  | CO3 |  | (10) |  |
| 4. | a) | Let x1(n) be {1, 1/2, 1/4, 1/8} and x2(n)= {1,1,1,1}. Use DIT FFT algorithm to compute DFT of the above sequences. Using the results compute the circular convolution of the above sequences. |  |  | CO3 |  | (12) |  |
|  | b) | The sequence x(n)= {1, 2, 3, 3, 2 , 1 , -1, -2, -3, 5, 6, -1, 2 ,0,2} is filtered through a filter with impulse response h(n)={1,-1}. compute the output of filter using an overlap-add technique use 5 point circular convolution. |  |  | CO2 |  | (08) |  |
|  |  | UNIT-III |  |  |  |  |  |  |
| 5. | a) | It is required to design an equiripple highpass filter of order N=64. The stop band ripple is to be no larger than δs=0.001 and the pass band ripple no larger than δp=0.01. If the pass band cut off frequency equal to p=0.72. What will be the stop band cut off frequency approximately equal to. |  |  | CO4 |  | (10) |  |
|  | b) | Use the window design method to design a minimum order filter with a stopband cut-off frequency ws=0.22π ,a passband frequency wp=0.28π and a stopband ripple δs=0.010 |  |  | CO4 |  | (10) |  |
| 6. | a) | Consider the following specifications for a filter │H(ejw)│≤ 0.01 0 ≤│w│≤ 0.2 0.95 ≤│H(ejw)│≤ 1.05 0.3π≤│w│≤ 0.7 │H(ejw)│≤ 0.02 0.8π ≤│w│≤ π - Design a linear phase FIR filter to meet these specifications using Hamming window. - Repeat part (i) using Hanning window. |  |  | CO4 |  | (10) |  |
|  | b) | Determine the impulse response h(n) of a filter having desired frequency response Hd () =e −j(N−1)2 for 0 ≤ ¦w¦ ≤ π2 0 for N=7 ,use frequency sampling approach |  |  | CO4 |  | (10) |  |
|  |  | UNIT-IV |  |  |  |  |  |  |
| 7. | a) | Design a digital filter using chebyshev approximation and bilinear transformation to meet the following specifications: Passband ripple=1dB for 0≤w≤0.15π Stop band attenuation ≥20dB for 0.45π ≤w≤π |  |  | CO3 |  | (12) |  |
|  | b) | Distinguish between FIR and IIR filter. |  |  | CO3 |  | (08) |  |
| 8. | a) | Explain the need of anti aliasing filters. |  |  | CO3 |  | (06) |  |
|  | b) | Design a Band pass analog filter with the following specifications. i. – 3.013dB lower and upper cut off frequencies of 50 Hz and 20kHz. ii. Stop band attenuation of at least 20 dB at 20 Hz and 45 kHz, iii. Monotonic frequency response. |  |  | CO3 |  | (14) |  |
|  |  | UNIT-V |  |  |  |  |  |  |
| 9. | a) | An LTI system is described by the following input-output relation. 2y (n) - y (n-2) + 4y (n-3) = 3x (n-2) Realize the system in the following forms: - Direct form I realization. - Direct form II realization. - Draw the transpose of the direct form II structure. |  |  | CO3 |  | (12) |  |
|  | b) | Realize the following system in: i) Direct form ii) Cascade form H(z)=1+ ¾ z -1 + 1⅞ z -2 + ¾ z -3 + z -4, |  |  | CO2 |  | (08) |  |
| 10. | a) |  | An LTI system is shown in figure below. Write the input- output relationship and Realize the system in the following forms: (i) Direct form I realization. (ii) Direct form II realization. (iii) Draw the transpose of the direct form II structure. | CO3 |  | (12) |  |  |
|  | b) | Realize a Linear phase FIR filter with the following impulse response. h(n)= δ(n) + ½ δ(n-1) - ¼ δ(n-2) + δ(n-4)+ ½ δ(n-3). Give necessary equations. |  |  | CO2 |  | (08) |  |

****************