RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS – JANUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE51/EE501 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With the help of block diagram describe digital signal processing system. List three applications. | CO1 | (06) |
|  | b) | Compute the DFT of the sequence defined by for (i) N=3 (ii) N=4 (iii) N=odd (iv) N=even. | CO1 | (06) |
|  | c) | Determine 8 point DFT of x(n)={1,1,1,1,1} and sketch magnitude and phase spectrum. | CO1 | (08) |
| 2. | a) | Let x(n)={2,1,1,0,1,2,0,2,3,4} with 10 point DFT X(k).Evaluate the following without explicitly computing DFT. (i)X(0), (ii)X(5), (iii) (iv) | CO1 | (06) |
|  | b) | Given, g(n)= {4, 3, 7, 5, 2, 1} where g(n) and h(n) are two sequences of length 6.They have 6 point DFTs G(k) and H(k) respectively. The DFT’s G(k) and H(k) are related by circular frequency shift as H(k)=G((k-3) . Determine h(n)without computing IDFT. | CO1 | (06) |
|  | c) | Consider the sequence x (n)=δ(n)+2δ(n-2)+δ(n-3) (i) Find the four point DFT of x(n). (ii) If y(n) is, the four point circular convolution of x (n) with itself. Find y(n) and the four point DFT Y(k). iii) With h(n) = δ(n)+δ(n-1)+2δ(n-3). Find the four point circular convolution of x(n) with h(n) using tabular array method. | CO1 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | The sequence x(n)= {1,2,3,4,4,2,3,1,1,2,4,4,1,1,5,7} is filtered through a filter whose impulse response is h(n)= {3, 2, 1, 1}. Compute the output of the filter y(n) using overlap save method. Use 9 point circular convolution. | CO2 | (10) |
|  | b) | Given, the sequence x(n)=sin((∏/2)n), 0 < n < 7, compute the 8-point DFT using DIT-FFT algorithm. | CO3 | (10) |
| 4. | a) | Find the sequence x(n) corresponding to the 8 point DFT X(k)= {4, 1-j2.41, 0, 1-j0.414, 0, 1+j0.414, 0, 1+j2.414} by using radix-2 DIF-FFT algorithm to compute IDFT. Draw the signal flow graph. | CO3 | (12) |
|  | b) | Let x1(n) be {1, 1/2, 1/4, 1/8} and x2(n) = {1,1,1,1}. Use DIT FFT algorithm to compute DFT of the above sequences. | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain linear phase FIR structures. Mention its advantages. Obtain linear phase realization of H(z)= 1+ (z-1)/4 + (z-2)/4 +z-3 | CO4 | (12) |
|  | b) | Realize the following FIR system function in cascade form. H(z)= (2/3)z +1+[3/4 (z-1)] | CO4 | (08) |
| 6. | a) | An LTI system is described by the following input-output relation. 2y (n) - y (n-2) – 4y (n-3) = 3x (n-2) Realize the system in the following forms: - Direct form I realization. - Direct form II realization. - Draw the transpose of the direct form II structure. | CO4 | (12) |
|  | b) | A discrete time IIR system H (z) is expressed as: H(z) = (8z3-4z2+11z-2)/ (z-0.25)(z2-z+0.5). Obtain the parallel form realization. | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Design a FIR low pass digital filter to be used in an ADC-H(Z)-DAC structure that will have a -3dB cut-off at 30 л rad/sec and an attenuation of 50dB at 45 л rad/sec. the filter is required to have a linear phase and the system will use a sampling rate of 100 samples/second. | CO4 | (O8) |
|  | b) | Design a low pass filter with frequency response Hd(ejω)=e-jωτ for ¦ω¦<π/4,and equals zero for π/4≤¦ω¦≤π using rectangular window WR(n)=1 for 0≤n≤4 and equals zero otherwise. Determine the coefficient of filter h(n) defined by h(n)=hd(n)WR(n). Also find the frequency response H(ω) of the resulting FIR filter. | CO4 | (12) |
| 8. | a) | Design an FIR low pass filter using rectangular window with pass band gain of unity, cut-off frequency of 1000Hz, sampling frequency of 5 kHz. Assume the length of the impulse response as 7. | CO4 | (O8) |
|  | b) | Determine the filter coefficients hd(n) of a filter having desired: frequency response of a low pass filter Hd()= e-j2ω for 0 for | CO4 | (O6) |
|  | c) | Compare the various design techniques of FIR filters. | CO4 | (O6) |
|  |  | UNIT- V |  |  |
| 9. | a) | For the transfer function of a discrete time causal system given below: H(z ) = - Find the difference equation realization. - Find the parallel realization. - Find the transpose structure of parallel form. | CO5 | (12) |
|  | b) | Obtain the direct form II and cascade realization for the system function given below: H(z)= | CO5 | (08) |
| 10. | a) | Consider an LTI system, initially described by the difference equation, y(n)= 0.25y(n-2)+x(n) - Determine the impulse response, h(n) of the system. - What is the response of the system to the input signal x(n)=[(0.5)n+(-0.5)n]u(n). - Determine the direct form-1,direct form-2,parallel form, cascade form realization for this system. | CO5 | (12) |
|  | b) | Obtain direct form and linear phase realization of: | CO5 | (08) |

****************