RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE51 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With the help of block diagram describe Digital Signal Processing System. Write advantages and disadvantages of DSP over ASP. | CO1 | (08) |
|  | b) | Consider the sequence x(n)=10(n)+3(n-1)+2(n-3)+(n-4)+2(n-5) - Compute the 6 point DFT of the sequence x(n). - Find the finite length sequence y(n) that has a 6 point DFT, Y(k)=W62kX(k); 0≤k≤5. - Find - Find X(K) e−j2πk3 | CO1 | (12) |
| 2. | a) | - Explain the following properties of DFT: - Circular symmetry ii) Circular convolution. | CO1 | (04) |
|  | b) | Determine 4 point convolution of x(n)=2n , 0 and h(n)=(-2)n , 0≤n≤2, using i) Stockham’s method , ii) graphical method | CO1 | (10) |
|  | c) | Find 8 point DFT of sequence of h(n)=(1/2) , -2 n = 0 otherwise | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | The sequence x(n)={3,-1,0,1,3,2,0,1,2,1} is filtered through a filter impulse response h(n)={1,1,1}. Compute the output of the filter using overlap save method. Select N appropriately. | CO2 | (10) |
|  | b) | Explain the DIT FFT Radix 2 algorithm to compute the DFT of signal and find DFT of x(n)= cos(n) using DIT FFT Radix 2 algorithm. | CO3 | (10) |
| 4. | a) | Compute the DFT of the sequence x(n) using DIT-FFT method and h(n) by DIF FFT method. Given x(n)= {1,1/2,1/4,1/8}, h(n)= {1,1,1,1}. Using the above results calculate the circular convolution of the two sequences. | CO2 | (12) |
|  | b) | The sequence x(n)= {2,1,-1,-2,-3,5,6,-1,2,0,2,1} is filtered through a filter whose impulse response is h(n)= {3, 2, 1}. Compute the output of the filter y(n) using overlap save method. Use 8 point circular convolution, | CO3 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Realize the following FIR system function in cascade form. H(z)= (2/3)z-2 +1+[3/4 (z-1)] | CO4 | (08) |
|  | b) | Explain linear phase FIR structures. Mention its advantages. Obtain linear phase realization of H(z)= 1+ (z-1)/4 + (z-2)/4 +z-3 | CO4 | (12) |
| 6. | a) | The transfer function of a discrete causal system is given as follows: H(z)= . Find the difference equation. Draw cascade realization. | CO4 | (12) |
|  | b) | A discrete time IIR system H (z) is expressed as: H(z) = (8z3-4z2+11z-2)/ (z-0.25)(z2-z+0.5). Obtain the parallel form realization. | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Design a bandpass linear phase FIR filter with cut-off frequencies of ωc1=1 rad/sample and ωc2=2 rad/sample. Obtain the impulse response through following window: w(n) = 1 for 0≤n≤6 = 0 elsewhere. | CO4 | (14) |
|  | b) | Explain different types of windows used in Fir filter design. | CO4 | (06) |
| 8. | a) | Consider the following specifications for a low pass filter Hd(ejw)= e-j3ω ;│ω│≤ 3π/4 = 0; 3π/4<│ ω │< π Determine the frequency response of the filter to meet these specifications using Hamming window with N =7. | CO4 | (14) |
|  | b) | Explain i)Gibb’s phenomenon ii) the need of anti-aliasing filters. | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Determine H(z) using impulse invariance method for following analog filter. | CO4 | (08) |
|  | b) | Use bilinear transformation to design a first order low pass Butterworth filter that has a 3dB cut-off frequency ωc =0.2. | CO4 | (12) |
| 10. | a) | Explain TMS320LF2407 digital signal controller. | CO5 | (06) |
|  | b) | Design a first order digital high pass Butterworth filter with cutoff frequency of 30Hz and sampling frequency 150 Hz by bilinear transformation. Explain the design wherever required. | CO4 | (14) |

****************