RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Signal Processing | Max. Marks | : | 100 |
| Course Code | : | EE51 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss briefly the applications of digital signal processing. | CO1 | (06) |
|  | b) | By means of DFT and IDFT, determine the response of the FIR filter with impulse response h(n)= (1,2,3) to the input sequence x(n)= (1,2,2,1). Choose N considering that FFT algorithm can used to for the efficient computation of DFT. Demonstrate the possible error in finding the output sequence if N is chosen as 4. | CO1 | (10) |
|  | c) | Let x(n) be a finite length sequence with X(k) = {0, 1+j, 1, 1-j}. Using circular time shift property find x((n-1))4. | CO2 | (04) |
| 2. | a) | Discuss any three properties of DFT. | CO2 | (06) |
|  | b) | The first five points of the 8 point DFT of a real valued sequence are {28, -j4.966, 4+4j, -4+1.66j, -4}. Determine remaining three points of the DFT. | CO2 | (06) |
|  | c) | Consider the sequence x(n)= u(n) - u(n-4), h(n) = u(n) - u(n-3). Evaluate circular convolution using DFT-IDFT method. Assume N=8. Plot the output. | CO2 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | Compute the circular convolution of the following sequences using DIF-FFT method. x(n)= {1,2,0,1}, h(n)= {1,3,3,1}. Show the intermediate calculation using butterfly structure. | CO3 | (10) |
|  | b) | Find the output y(n) of a filter whose impulse response is h(n)={1,1,1}and the input signal to the filter is x(n)={3,-1,0,1,3,2,0,1,2,1}using overlap save method. | CO2 | (10) |
| 4. | a) | Develop DIT FFT Radix 2 algorithm to compute the DFT X(k) of signal x(n) and hence find the 4 point DFT of the sequence . | CO3 | (12) |
|  | b) | The sequence x(n)= {1,2,-1,2,3,-2,-3,-1,1,1,2,-1} is filtered through a filter whose impulse response is h(n)= {1, 2, 3}. Compute the output of the filter y(n) using overlap add method. Use 6 point circular convolution. | CO1 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Obtain DF-I, DF-II and cascade structure of the system represented by the difference equation. | CO4 | (10) |
|  | b) | Realize a linear phase FIR filter with the impulse response . Give necessary equations. | CO4 | (10) |
| 6. | a) | Discuss any two structures of realizing an IIR system. | CO4 | (12) |
|  | b) | Obtain a linear phase FIR structure having even value of impulse samples ‘M’. | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Design a FIR low pass digital filter whose desired frequency response is given as Hd ()= e-j3ω for 0 otherwise. Given ωc=1 rad/sample. Use rectangular window. | CO4 | (10) |
|  | b) | Design a low pass filter with frequency response Hd(ejω)=e-jωτ for ¦ω¦<π/4,and equals zero for π/4≤¦ω¦≤πusing rectangular window WR(n)=1 for 0≤n≤4 and equals zero otherwise. Determine the coefficient of filter h(n) defined by h(n)=hd(n)WR(n). | CO4 | (10) |
| 8. | a) | Determine the impulse response h(n) of a filter having desired frequency response Hd(ejω)=e-jωτ for 0≤¦ω¦≤/2,and equals zero for /2≤¦ω¦≤π. Assume the length M=7. Use frequency sampling approach. | CO4 | (10) |
|  | b) | Explain the FIR filter design using window method. Also explain various windows used in Filter design. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Design a digital filter using chebyshev approximation and bilinear transformation to meet the following specifications: Passbandripple=1dB for 0≤w≤0.15 Stopband attenuation ≥20dB for 0.45 ≤w≤ | CO4 | (10) |
|  | b) | Describe the architecture of C2xx DSP CPU. | CO5 | (10) |
| 10. | a) | Explain TMS320LF2407 digital signal controller. | CO5 | (06) |
|  | b) | Design a Band pass analog filter with the following specifications. i. – 3.013dB lower and upper cut off frequencies of 50 Hz and 20kHz. ii. Stop band attenuation of at least 20 dB at 20 Hz and 45 kHz, iii. Monotonic frequency response. | CO4 | (14) |

****************