RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY/AUGUST 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE51/EE501 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the various operations involved in Digital Signal Processing. | CO1 | (04) |
|  | b) | Compute N point DFT of x(n)=an , 0 ≤ n ≤ N-1. | CO3 | (06) |
|  | c) | Given two sequences x1(n) = (1, 2, 3, 1), x2(n) = (4,3,2,2). Find x3 (n) such that X3(k) = X1(k) X2(k) using any two methods. | CO2 | (10) |
| 2. | a) | Consider the sequence x(n)= u(n) - u(n-4), h(n) = u(n) - u(n-3). Evaluate circular convolution using DFT-IDFT method. Assume N=8. Plot the output. | CO2 | (10) |
|  | b) | The first five points of the 8 point DFT of a real valued sequence are {28, -j4.966, 4+4j, -4+1.66j, -4}. Determine remaining three points of the DFT. | CO2 | (04) |
|  | c) | Write the circular time shift and circular frequency shift properties of DFT. | CO2 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | The sequence x(n)={1,1,1,1,3,1,1,4,2,1,1,3,1,1} is filtered through a filter impulse response h(n)={1,-1}. Compute the output of the filter using overlap add method. Use 5-point circular convolution. | CO2 | (10) |
|  | b) | Develop DIF FFT Radix 2 algorithm to compute the DFT X(k) of signal x(n). | CO3 | (10) |
| 4. | a) | Compute the circular convolution of the following sequences using DIT-FFT method. x(n)= {1,2,0,1}, h(n)= {1,3,3,1}. Show the intermediate calculation using butterfly structure. | CO2 | (10) |
|  | b) | The sequence x(n)= {1,2,-1,2,3,-2,-3,-1,1,1,2,-1} is filtered through a filter whose impulse response is h(n)= {1, 2, 3}. Compute the output of the filter y(n) using overlap save method. Use 6 point circular convolution. | CO3 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | We would like to filter an analog signal xa(t) with an analog low pass filter that has a cut-off frequency fc=2kHz , a transition width ∆f=500 Hz & stopband attenuation of 50dB. This filter is to be implemented digitally, as illustrated in the fig below. Design a digital filter to meet the analog filter specifications with a sampling frequency fs=10k Hz. | CO3 | (08) |
|  | b) | Use the window design method to design a minimum order filter with a stopband cut-off frequency ws=0.22 ,a passband frequency wp=0.28 and a stopband ripple δs=0.010. | CO3 | (08) |
|  | c) | Explain Gibb’s phenomenon. | CO3 | (04) |
| 6. | a) | Consider the following specifications for a filter: H(ejw)≤ 0.01 0 ≤w≤ 0.2π 0.95 ≤H(ejw)≤ 1.05 0.3π≤w≤ 0.7π H(ejw)≤ 0.02 0.8π ≤w≤ π - Design a linear phase FIR filter to meet these specifications using Hamming window. - Repeat part (i) using Hanning window. | CO3 | (10) |
|  | b) | Determine the impulse response h(n) of a filter having desired frequency response: Hd()= for 0 for N=7 ,use frequency sampling approach. | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Distinguish between FIR and IIR filter. | CO4 | (08) |
|  | b) | Design an analog band pass filter to meet the following frequency -domain specifications: i) A -3.0103dB upper and lower cutoff frequency of 50 hertz and 20 kilohertz ii) a stop band attenuation of at least 20 db at 20 hertz and 45 kilohertz ,and iii) a monotonic frequency response. | CO4 | (12) |
| 8. | a) | Explain impulse invariance method of transforming analog filter into an equivalent digital filters. | CO4 | (06) |
|  | b) | A second order continuous time filter has the system function: where a<0 and b<0 are real. (i)Determine the locations of the poles and zeroes of H(z) if the filter is designed using the bilinear transformation with Ts=2. | CO4 | (06) |
|  | c) | Design a digital low pass filter using Chebyshev filter design procedure that meets the following specifications: Pass band magnitude characteristics that is constant to within 1dB for frequencies below w=0.2π and stop band attenuation of atleast 15dB for frequencies between w=0.3π and π use Bilinear transformation. | CO4 | (08) |
|  |  | UNIT- V |  |  |
| 9. | a) | Design the Chebyshev low pass filter using bilinear transformations with frequency response 0.8 ≤ H(ω)≤1 for 0≤¦ω¦≤0.2π H(ω)≤0.2 for 0.32π ≤¦ω¦≤π | CO4 | (15) |
|  | b) | Compare FIR and IIR filters. | CO4 | (05) |
| 10. | a) | Design a digital low pass filter using Chebyshev filter design procedure that meets the following specifications: Pass band magnitude characteristics that is constant to within 1dB for frequencies below ω=0.2π and stop band attenuation of at least 15dB for frequencies between ω=0.3∏ and ∏. Use Bilinear transformation. | CO4 | (14) |
|  | b) | Explain how bilinear transformation removes the drawback of impulse invariant transform? | CO4 | (06) |

****************