| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)

Accredited by NBA & NAAC with ‘A+’ Grade

EXAMINATIONS SEPTEMBER /OCTOBER 2021

SUPPLEMENTARY SEMESTER / GRADE IMPROVEMENT/ RE –REGISTERED CANDIDATES

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Signal Processing | Max. Marks | : | 100 |
| Course Code | : | EE61 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer any five full questions.

| 1. | a) | Discuss briefly various applications of digital signal processing. | CO1 | (04) |
| --- | --- | --- | --- | --- |
|  | b) | Determine the response of the FIR filter whose unit sample response is given as h(n) = (1,2) when the input applied is x(n)= (2,1). Use circular convolution and verify the result using linear convolution. | CO1, CO2, CO3 | (08) |
|  | c) | Given two sequences x1(n) = (2, 1, 2, 1), x2(n) = (1,2,3,4). Find x3 (n) such that X3(k) = X1(k) X2(k). | CO2 | (08) |
| 2. | a) | Perform the circular convolution of the following two sequences using graphical method and DFT-IDFT method and hence verify the circular convolution property.Given x1(n) = (2, 1, 2, 1), x2(n) = (1,2,3,4). | CO2 | (10) |
|  | b) | The first five points of the 8 point DFT of a real valued sequence are {0.25, 0.125-j0.3018, 0, 0.125-j0.0518, 0}. Determine remaining three points of the DFT. | CO2 | (06) |
|  | c) | Find the 4 point IDFT of the sequence X(k)= (4, -j2,0,j2). | CO2 | (04) |
| 3. | a) | Compute the 8 point DFT of the sequence: x(n)= {1/2,1/2,1/2,1/2, 0,0,0,0}using both DITFFT and DIFFFT algorithms. Mark all the intermediate quantities on the signal flow diagram. | CO2 | (14) |
|  | b) | Explain the computational complexity of DITFFT algorithm compared to direct method with necessary calculations. | CO2 | (06) |
| 4. | a) | Develop DIF FFT Radix 2 algorithm to compute the DFT X(k) of signal x(n). | CO2,CO3 | (10) |
|  | b) | The sequence x(n)= {1,2, -1,2,3, -2, -3, -1,1,1,2, -1} is filtered through a filter whose impulse response is h(n)= {1, 2, 3}. Compute the output of the filter y(n) usingoverlap add method. Use 6 point circular convolution, | CO1,CO2,CO3 | (10) |
| 5. | a) | An LTI system is described by the following input-output relation. 2y (n) - y (n-2) +3y (n-3) = 4x (n-2) Realize the system in the following forms: - Direct form I realization. - Direct form II realization. Draw the transpose of the direct form II structure. | CO3 | (12) |
|  | b) | Realize the following system in: i) Direct form ii) Cascade form H(z)=1+ ¾ z -1 + ⅞ z -2 + ¾ z -3 + z -4, | CO2 | (08) |
| 6. | a) | An LTI system is shown in figure below. Write the input- output relationship and Realize the system in the following forms: - Direct form I realization. - Direct form II realization. - Draw the transpose of the direct form II structure. | CO3 | (12) |
|  | b) | Realize a Linear phase FIR filter with the following impulse response. h(n)= δ(n) + ½ δ(n-1) - ¼ δ(n-2) + δ(n-4)+ ½ δ(n-3). Give necessary equations. | CO2 | (08) |
| 7. | a) | A filter is to be designed with the following desired frequency response: Hd(w)= { 0, -π/4 ≤ w ≤ π/4 { e-j2w , π/4 ≤│w│ ≤ π, Find the frequency response of the FIR filter designed using a Hamming window. | CO3 | (08) |
|  | b) | Design a 17-tap linear-phase FIR filter with a cutoff frequency wc = π/2. The design is to be done based on frequency sampling technique. (Consider low pass filter). | CO3 | (08) |
|  | c) | Explain Gibb’s phenomenon. | CO3 | (04) |
| 8. | a) | Obtain the coefficients of an FIR filter to meet the specifications given below using the window method. 0.99 ≤│Hd(w)│ ≤ 1.01 , 0 ≤│w│ ≤ 0.375 π │Hd(w)│ ≤ 0.01 , 0.5π ≤│w│ ≤ π. | CO4 | (10) |
|  | b) | To design an equiripple high pass filter of order N=64, the stopband ripple is to be no larger than δs =0.001 & the passband ripple no larger than δp =0.01. If a passband cut-off frequency is assumed to be equal to wp=0.72π, what will be the approximate value of stopband cut-off frequency? | CO4 | (10) |
| 9. | a) | Design a digital filter using chebyshev approximation and bilinear transformation to meet the following specifications: Passbandripple=1dB for 0≤w≤0.15π Stopband attenuation ≥20dB for 0.45π ≤w≤π | CO4 | (10) |
|  | b) | Describe the architecture of C2xx DSP CPU. | CO5 | (10) |
| 10. | a) | Write a note on TMS320LF2407 digital signal controller. | CO5 | (06) |
|  | b) | Design a Band pass analog filter with the following specifications. i. – 3.013dB lower and upper cut off frequencies of 50 Hz and 20kHz. ii. Stop band attenuation of at least 20 dB at 20 Hz and 45 kHz, iii. Monotonic frequency response. | CO4 | (14) |

****************