RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

MAKEUP EXAMINATIONS – MAY/JUNE 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Generation Economics and Reliability Aspects of Power Systems | Max. Marks | : | 100 |
| Subject Code | : | EEPE42 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Missing data may be assumed suitably.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain the functions of the following with respect to nuclear power plant (i) Control rod (ii) reflector and (iii) moderator. | CO2 | (06) |
|  | b) | Draw the line diagram of a thermal power plant and discuss its operation. | CO2 | (10) |
|  | c) | Explain the functions of (i) surge tank and (ii) spillways. | CO2 | (04) |
| 2. | a) | Explain the necessity of feed water treatment in a steam power plant. | CO2 | (06) |
|  | b) | Draw the line diagram of a diesel power plant and explain its operation. | CO2 | (10) |
|  | c) | List the various criteria for selection of site for thermal power plant. | CO2 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | List the causes for the replacement of the electrical equipments. | CO1 | (04) |
|  | b) | Maximum demand of consumer is 20Amps at 220Volt,his total energy consumption is 8760kWh.If the energy is charged at 20 Paise for 500 Hours for use of maximum demand per annum plus 10 paise per unit for additional units ,Calculate the annual bill and Equivalent flat rate? | CO1 | (08) |
|  | c) | On the basis of annual operation the use factor and capacity factor are 0.5 and 0.4 .Find the no of hours of its operation during the year? | CO1 | (05) |
|  | d) | State wheather following statements are true or False i)Contract labour salary is Fixed cost .True/False ii)Improvement in power factor yields to less consumption of energy True/False iii)Demand factor is the ratio of maximum demand to the connected load. True/False | CO1 | (03) |
| 4. | a) | Discuss the evolution of energy market. | CO1 | (04) |
|  | b) | Estimate generating cost/kWh of generating station with following data Max demand =50MW,Annual load factor=40% Capital cost is 1.2 crores , annual cost of wages and taxes=Rs 4lakhs,Maintainance=1.0paise per kWh. Interest and depreciation is 11% | CO1 | (08) |
|  | c) | A power station has a maximum demand of 100MW has a load factor of 30 %.Steam or hydro station is capable of supplying the whole load .Compare overall cost per kWH generated with following data: Steam; Hydro. Capital cost/kWh; Rs 1250; Rs 2500. Interest and Depreciation; 12%; 10%. Operating cost/kWh; 5 paise; 1.5 paise. Transmission Cost; negligible; 0.2paise. | CO1 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Consider a system containing 5 generating units each of 40MW and having a forced outage rate of 0.01. Compute the capacity outage table. | CO4 | (05) |
|  | b) | Explain the different functional zones and hierarchical levels in reliability. | CO3 | (07) |
|  | c) | Explain the effects of rounding and load forecast uncertainty on the adequacy evaluation of generation system. | CO3 | (08) |
| 6. | a) | Explain briefly about the reliability test system. | CO3 | (05) |
|  | b) | Calculate LOLE for the following data for the system load: System data: Unit No; Capacity (MW); FOR. 1; 25; 0.02. 2; 25; 0.02. 3; 50; 0.02. Load Data: Daily Peak Load(MW); 57; 52; 46; 41; 34. No. of occurences; 12; 83; 107; 116; 47. | CO4 | (10) |
|  | c) | What is a derated state? Explain its effect in the reliability evaluation. | CO3 | (05) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Why load curtailment techniques are necessary? Explain its various philosophies. | CO3 | (10) |
|  | b) | Explain Monte Carlo Simulation with respect to convergence and computing time. | CO3 | (10) |
| 8. | a) | Differentiate between ENEL and U of S approach. | CO3 | (10) |
|  | b) | Explain how the station originated outages is taken into account during composite adequacy evaluation. | CO3 | (05) |
|  | c) | Explain about the choice of appropriate network solution technique used in adequacy evaluation. | CO3 | (05) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the concept of load curtailment technique. | CO4 | (06) |
|  | b) | With a flowchart explain contingency enumeration approach. | CO4 | (05) |
|  | c) | List the advantages of DC Load flow. | CO4 | (05) |
|  | d) | What factors contribute to outages in Power systems. | CO4 | (04) |
| 10. | a) | what factors plays important role in assessing Commercial customer interruptions. | CO5 | (05) |
|  | b) | What are the assumptions considered to analyze the interruption scenarios. | CO5 | (05) |
|  | c) | Find SAIFI,SAIDI,CAIDI,ASAI for the following set of customer data: Line; Failure rate; Repair rate; No of customers; Avg load demand. A; 0.20; 6.00; 200; 1000. B; 0.10; 5.00; 150; 700. C; 0.15; 8.0; 100; 400. | CO5 | (10) |

****************