RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS –  MAY/JUNE 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Generation Economics & Reliability Aspects of Power Systems | Max. Marks | : | 100 |
| Subject Code | : | EEPE42 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Missing data may be assumed suitably

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain with a neat sketch the various parts of a nuclear reactor. | CO2 | (10) |
|  | b) | Explain the following with suitable diagrams: (i)Surge tank (ii)lubricating system in a diesel power plant (iii) coal handling system in a steam station | CO2 | (10) |
| 2. | a) | Explain the following with respect to steam power plant - Air preheater (ii) Super heater (iii) Economiser | CO2 | (09) |
|  | b) | Discuss about the waste disposal in a nuclear power plant. | CO2 | (05) |
|  | c) | Discuss the factors to be considered in the choice of site for a nuclear power plant. | CO2 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Two systems of tariff are available for a factory working 8 hours a day for 300 working days in a year (i)High voltage supply at 5paise per unit plus Rs 4.50 per month per kVA of maximum demand (ii)Low Voltage supply at Rs 5 per month per kVA of maximum demand plus 5.5 paise per unit The factory has an average load of 200kW at 0.8pf and a maximum demand of 250kW at the same powerfactor. The high voltage equipment costs Rs 50per kVA and the losses can be taken as 4%. Interest and depreciation charges are 12%. Calculate the difference in the annual costs between the two systems. | CO1 | (10) |
|  | b) | Discuss the advantages of interconnected system. | CO1 | (05) |
|  | c) | Discuss about the energy market in India. | CO1 | (05) |
| 4. | a) | A 20,000kW generating station has the following generating cost: capital cost =Rs 65x105; Annual Taxation=Rs 1.5x105; Annual salaries and wages=Rs 6x105. The calorific value of the coal burnt is 54.5x105cal/kg. The cost of fuel is Rs 55 per tonne.Assume plant heat rate as 29x105cal/kWh at 100% capacity factor and 38x105cal/kWh at 60% capacity factor. Calculate the generating cost at 60% and 100% capacity factor. Interest and depreciation is 10%. | CO1 | (10) |
|  | b) | A steam station is erected at a cost of Rs 2x108. Salvage value is 15% . Life of the plant is 25 years. Sinking fund is 8%. Find depreciation value & value of the equipment after 10 years by (i) straight line method (ii) Diminishing value method (iii) Sinking Fund method. | CO1 | (10) |
|  |  | UNIT-III |  |  |
| 5. | a) | Explain the necessity of reliability evaluation. | CO3 | (06) |
|  | b) | List the factors which increases the reliability of power system. | CO3 | (05) |
|  | c) | A power plant has four units of 100MW of capacity with repair rate=0.49f/year and failure rate=0.01f/year. Prepare the capacity outage probability table? | CO3 | (09) |
| 6. | a) | Explain the reliability curve with cost and interruption concepts. | CO3 | (05) |
|  | b) | Enumerate the difference between Analytical & probabilistic technique. | CO3 | (06) |
|  | c) | For the following LDC curve shown in Fig.6.(c) calculate the EIR with system having three identical units of having capacity 100 MW with FOR 0.02? Fig.6.(c) | CO3 | (09) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain the various load curtailment passes to be considered while evaluating reliability of composite system. | CO3 | (10) |
|  | b) | Differentiate between ENEL and U of S approach. | CO3 | (10) |
| 8. | a) | Write short notes on (i) station originated outages and (ii) effect of load curtailment passes. | CO3 | (10) |
|  | b) | Discuss on the appropriate load flow techniques to be used in composite system evaluation. | CO3 | (05) |
|  | c) | Explain the various factors to be considered while selecting the appropriate contingency level. | CO3 | (05) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain how the interruption cost of residential customer varies with different factors. | CO5 | (10) |
|  | b) | Define the terms: SAIFI, CAIDI,SAIDI,CAIFI and ASAI. | CO4 | (10) |
| 10. | a) | write short notes on: (i) customer damage function and (ii) interruption energy assessment rate. | CO5 | (10) |
|  | b) | Explain how the interruption cost for industrial customers is evaluated. | CO5 | (10) |

****************