RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS – JANUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Generation, Economics and Reliability Aspects of Power Systems | Max. Marks | : | 100 |
| Course Code | : | EEE20 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the impact of power generation in steam power plant on environment and the measures to be taken for minimizing environment effects. | CO1 | (08) |
|  | b) | Explain with neat sketch the following with respect to hydroelectric plant: (i) Surge tank and (ii) Draft tube. | CO1 | (08) |
|  | c) | What are the advantages of diesel power station? | CO1 | (04) |
| 2. | a) | Explain briefly the functions of the following in a thermal power plant: (i) Super heaters and Re-heaters (ii) Condensers (iii) Economizer (iv) Cooling towers. | CO1 | (08) |
|  | b) | What are the factors taken into account while selecting the site for a nuclear power station? | CO1 | (08) |
|  | c) | Explain the function of lubricating system and exhaust system in diesel power station. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | Explain briefly two part tariff, power factor tariff and maximum demand tariff. | CO2 | (06) |
|  | b) | A 300MW thermal power station is to supply power to a system having maximum and minimum demand of 240MW and 180MW respectively in a year. Assuming the load duration curve to be a straight line, estimate the load factor and capacity factor. | CO2 | (06) |
|  | c) | A generating station has a maximum demand of 50MW. Calculate the cost per kWh delivered from the following data: (i) Capital cost = Rs 95x105 (ii) annual cost of fuel + oil = Rs 9x105 (iii) taxes, wages and salaries= Rs6x105 and rate of interest and deprecation is 10%. Annual load factor is 50%. | CO2 | (08) |
| 4. | a) | Define power factor. What are the effects of low P.F? What are the methods of improving P.F? | CO2 | (06) |
|  | b) | Define the following terms: (i) Demand factor (ii) Load factor (iii) Diversity factor (iv) Plant use factor and (v) Load curve duration. | CO2 | (06) |
|  | c) | The equipment in a power station costs Rs 15,60,000 and has a salvage value of Rs 60,000 at the end of 25 years. Determine the depreciated value of the equipment at the end of 20 years by the following methods: (i) Straight line method (ii) Diminishing value method and (iii) Sinking fund method at 5% compound interest annually. | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | How the hierarchical levels in the power system are classified. | CO3 | (06) |
|  | b) | List the factors which increases the reliability of the power systems. | CO3 | (06) |
|  | c) | A power plant having four systems having two units a and b its state table is given below ,Draw the state transition diagram (u-up and D-down) µ-repair rate λ-failure rate System; Units-a; Unit-b. S1; U; D. S2; D; U. S3; U; U. S4; U; D. | CO3 | (08) |
| 6. | a) | Discuss the adequacy evolution of HL-I in reliability domain. | CO3 | (05) |
|  | b) | Discuss why probabilistic technique is better than analytic technique. | CO3 | (06) |
|  | c) | Explain cost reliability curve. | CO3 | (06) |
|  | d) | Indirect cost comprises major contribution than the direct cost in blackouts, explain. | CO3 | (03) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Define the terms LOLE,LOEE, EIR and EIU. | CO4 | (08) |
|  | b) | Classify maintenance in reliability domain. | CO4 | (06) |
|  | c) | A power plant consists of 5 Units with 3x30MW with Forced outage rate of 0.03 and 2x20MW with forced outage rate of 0.02, Calculate the cumulative outage probability table and combined probability table. | CO4 | (06) |
| 8. | a) | Differentiate between AC and DC load flow analysis. | CO4 | (08) |
|  | b) | A system having three units their capacities ,their failure rate and repair rate are given in the table below .Prepare a capacity outage probability table: Unit; Capacity(Mw); Failure rate; Repair rate. 1; 50; 1; 99. 2; 100; 1; 49. 3; 200; 1.1; 72.23. | CO4 | (08) |
|  | c) | List the assumptions taken in station originated outages. | CO4 | (04) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the terms SAIFI , SAIDI and ASAI. | CO5 | (06) |
|  | b) | Discuss the factors taken into consideration for accessing the interruption cost in the case of industrial customers. | CO5 | (06) |
|  | c) | In a feeder total no of customers served is 50,000, total no of customers affected is 16,000 , total no of customer interruption is 17,000,total no of customer interruption duration is 1,24,200 minutes. Calculate SAIFI, SAIDI CAIFI and CAIDI. | CO5 | (08) |
| 10. | a) | Differentiate between Smart Grid and traditional Grid. | CO5 | (06) |
|  | b) | Discuss the functional units of smart grid architecture. | CO5 | (08) |
|  | c) | List the conditions necessary for grid integration. | CO5 | (06) |

****************