| M S RAMAIAH INSTITUTE OF TECHNOLOGY USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(AUTONOMOUS INSTITUTE, AFFILIATED TO VTU)

BANGALORE – 560 054

SEMESTER END EXAMINATIONS – MAY/JUNE 2016

| Course & Branch | : | B.E. – Electrical & Electronics Engg. | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EEPE43 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT -I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With reference to the breakdown of gaseous dielectric, define the following terms. - Mobility of Ions - Elastic collisions - Mean free path - Collision of cross section - Self-sustained discharge. | C01 | (10) |
|  | b) | Define Townsend’s first and second ionization coefficients and derive an expression for the current growth in a gas discharge due to secondary ionization. | C01 | (10) |
| 2. | a) | Discuss with necessary equations, breakdown in Non-uniform fields. | C01 | (06) |
|  | b) | Determine the breakdown voltage of air gap 2mm and 5mm lengths under uniform field and standard atmospheric conditions. Also find the voltage if atmospheric pressure is 750mm of Hg and temperature 35oC. | C01 | (06) |
|  | c) | With relevant equations, explain formation of avalanche and leading to streamer and further, causing the breakdown of gases. | C01 | (08) |
|  |  | UNIT – II |  |  |
| 3. | a) | What is condition for breakdown of solid dielectric due to electromechanical? Derive an expression for maximum electrical stress that a solid dielectric can withstand. | C01 | (08) |
|  | b) | Mention the important properties of Transformer oil. | C01 | (04) |
|  | c) | What is partial discharge and explain with necessary waveform and equation, breakdown due to internal discharges. | C01 | (08) |
| 4. | a) | An air bubble of 0.25 mm thickness is embedded in a solid insulation of 2cm thickness having relative dielectric constant of 3. The voltage applied across the solid dielectric is 50kV (rms), 50Hz. Calculate the voltage at which internal discharge occurs and also voltage across the air bubble. | C01 | (08) |
|  | b) | Discuss the thermal breakdown in solid dielectric with necessary equations. | C01 | (06) |
|  | c) | How treeing and tracking causes the breakdown in solid dielectric. | C01 | (06) |
|  |  | UNIT – III |  |  |
| 5. | a) | Why high frequency AC voltages are to be generated? Explain with neat sketch generation of the same. | C02 | (08) |
|  | b) | What is cascading of Transformers? With neat sketch explain functioning of cascaded transformer for HVAC of power frequency. | C02 | (08) |
|  | c) | Discuss the limitations of half wave rectifier in generating HVDC. | C02 | (04) |
| 6. | a) | What is voltage doubler circuit? Explain its working with neat sketch. | C02 | (08) |
|  | b) | Mention the advantages of High Frequency Transformers . | C02 | (04) |
|  | c) | Discuss the working of Electrostatic generator with neat sketch. | C02 | (08) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Define standard Lightning and Switching impulse voltages. | CO2 | (04) |
|  | b) | For the single stage impulse generator shown in the Fig.7(b), obtain an expression for the output. Fig.7(b) | CO2 | (10) |
|  | c) | A 5-stage impulse generator has capacitors each rated at 0.05µF, 20kV. The capacitance of the test specimen is 1000pF. Determine the wave front, wave tail resistances and efficiency to produce a 1.2/50µs impulse wave using approximate formulae. | CO2 | (06) |
| 8. | a) | Define standard impulse current wave. | CO3 | (02) |
|  | b) | How are rectangular current pulses generated for testing purposes? How is their time duration controlled? | CO3 | (06) |
|  | c) | An impulse current generator is rated for 60kW-s. The parameters of the circuit are C=53µF, L=1.47µH and the dynamic resistance=0.0156 ohm. Determine the peak value of the current and the time-to-front and the time-to-tail of the current waveform. | CO3 | (06) |
|  | d) | What is a trigatron gap? Explain it functions and operation. | CO2 | (06) |
|  |  | UNIT – V |  |  |
| 9. | a) | Describe the principle, construction and working of generating voltmeter used for measuring high DC voltages. | CO4 | (10) |
|  | b) | Explain the principle and construction of an electrostatic voltmeter for very high voltage measurements. List the advantages and disadvantages. | CO4 | (10) |
| 10. | a) | Discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO4 | (08) |
|  | b) | Explain the resistance divider for impulse voltage measurement with impedance matching at the divider end. | CO4 | (12) |

********************