| M S RAMAIAH INSTITUTE OF TECHNOLOGY USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(AUTONOMOUS INSTITUTE, AFFILIATED TO VTU)

BANGALORE – 560 054

MAKEUP EXAMINATIONS – JULY 2016

| Course & Branch | : | B.E. – Electrical & Electronics Engg. | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EEPE43 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain different ionization processes in gases. | CO1 | (12) |
|  | b) | Derive an expression for the current growth in gases considering the secondary ionization process. | CO1 | (08) |
| 2. | a) | What are electronegative gases? Derive the criterion for breakdown in electronegative gases. | CO1 | (08) |
|  | b) | In a parallel wire system placed in air, the radius of wire is 0.1 inch. Assume, smooth surface for the wire, pressure 910 mbar and temperature 30oC. Calculate the corona inception gradient. | CO1 | (06) |
|  | c) | Determine the breakdown voltage for air gaps of 2mm and 5mm lengths under uniform field and standard atmospheric conditions. Also find the voltage if atmospheric pressure is 750mm of Hg and temperature 35oC. | CO1 | (06) |
|  |  | UNIT – II |  |  |
| 3. | a) | List the properties of Transformer oil. | CO1 | (04) |
|  | b) | With necessary equations, explain the suspended particle theory. | CO1 | (10) |
|  | c) | What are commercial liquid dielectrics, and how are they different from pure liquid dielectrics? | CO1 | (06) |
| 4. | a) | Discuss the thermal breakdown in solid dielectrics. | CO1 | (08) |
|  | b) | How does the ‘internal discharge’ phenomenon lead to breakdown in solid dielectrics? | CO1 | (08) |
|  | c) | A 0.1cm thick solid specimen of dielectric has a dielectric constant of 4.2, and tanδ=0.001 at a frequency of 50Hz. If it is subjected to an alternating voltage of 5kV, calculate the heat generated in the specimen due to the dielectric loss. | CO1 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain the working principle of resonant transformer and how is it advantageous over the cascade-connected transformers? | C02 | (08) |
|  | b) | With neat sketch, explain construction and working of Van De Graff Generator. | C02 | (08) |
|  | c) | Explain the working principle of Electrostatic generators. | C02 | (04) |
| 6. | a) | A ten stage Cockroft-Walton circuit has all capacitors of 0.08µF. The secondary voltage of the supply transformer is 150kV at a frequency of 100Hz. If the load current is 1mA, determine (i)voltage regulation (ii)the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | C02 | (08) |
|  | b) | Mention the advantages of High Frequency Transformers . | C02 | (04) |
|  | c) | Explain half wave rectifier for the generation of HVDC and list the limitations. | C02 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Discuss the limitations of single stage impulse generator. | C02 | (04) |
|  | b) | Draw a neat sketch of Marx generator and explain its important components. | C02 | (10) |
|  | c) | Draw impulse current waveform and explain the basic circuit for generating impulse currents. | C03 | (06) |
| 8. | a) | Draw the schematic setup for producing the rectangular current pulses and explain its working. | C03 | (08) |
|  | b) | What is Tigatron gap arrangement? Explain how its helps multistage impulse generator in generating the impulses. | C03 | (08) |
|  | c) | List the important components of an impulse current generator. | C03 | (04) |
|  |  | UNIT – V |  |  |
| 9. | a) | Explain Chubb Fortscue method for measurement of high voltage AC peak. | C04 | (08) |
|  | b) | Describe the construction and working of Electrostatic voltmeter for the measurement of high voltage. | C04 | (08) |
|  | c) | A generating voltmeter has to be designed so that it can have a range from 20 to 200 kV dc. If the indicating meter reads a minimum current of 2µA and maximum current of 25µA, what should the capacitance of the generating voltmeter be? | C04 | (04) |
| 10. | a) | Explain different voltage divider circuits for the measurement of high voltages. | C04 | (10) |
|  | b) | Explain the sphere gap arrangement for the measurement of high voltages and discuss the factors affecting the measurement. | C04 | (10) |

**********************