RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

MAKEUP EXAMINATIONS – MAY/JUNE 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EEPE43 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Define Townsend’s first and second ionization coefficients. How is the condition for breakdown obtained in a Townsend discharge? | CO1 | (08) |
|  | b) | In an experiment in a certain gas it was found that the steady state current is 5.5 x 10-8 A at 8 kV at a distance of 0.4cm between the plane electrodes. Keeping the field constant and reducing the distance to 0.1cm results in a current of 5.5 x 10-9 A. Calculate Townsend’s primary ionization coefficient. | CO1 | (04) |
|  | c) | What are electronegative gases? Discuss with necessary equations why these gases offer higher dielectric strength. | CO1 | (08) |
| 2. | a) | With relevant equations, discuss the formation of avalanche leading to streamer causing the breakdown of gases. | CO1 | (07) |
|  | b) | Determine the breakdown voltage of air gap of 2mm and 5mm lengths under uniform field and standard atmospheric conditions. Also find the voltage if atmospheric pressure is 750mm of Hg and temperature 35oC. | CO1 | (06) |
|  | c) | What is time lag? Explain it with a sketch. Discuss the factors on which time lag for the breakdown of a gaseous dielectric depend. | CO1 | (07) |
|  |  | UNIT- II |  |  |
| 3. | a) | List the properties of Transformer oil. | CO1 | (04) |
|  | b) | What are commercial liquid dielectrics, and how are they different from pure liquid dielectrics? | CO1 | (06) |
|  | c) | Define the intrinsic strength of a solid dielectric. | CO1 | (02) |
|  | d) | With neat figure explain the Avalanche breakdown in solid dielectrics. | CO1 | (08) |
| 4. | a) | Discuss Electromechanical breakdown in solid dielectrics. | CO1 | (08) |
|  | b) | How does the ‘internal discharge’ phenomenon lead to breakdown in solid dielectrics? | CO1 | (08) |
|  | c) | A 0.0394 inch thick solid specimen of dielectric has a dielectric constant of 4.2, and tanδ=0.001 at a frequency of 50Hz. If it is subjected to an alternating voltage of 5kV, calculate the heat generated in the specimen due to the dielectric loss. | CO1 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | What is cascading of transformer? Explain the working of it. | CO2 | (08) |
|  | b) | What is the need of generation of high frequency AC high voltages? Explain the method of generating it. | CO2 | (08) |
|  | c) | Mention the advantages of High Frequency Transformers. | CO2 | (04) |
| 6. | a) | For a high voltage generator using Cockcroft-Walton circuit with 4 stages and AC input voltage of rms 70.71kV, 200Hz, the load current of 4mA and each stage capacitance of 0.02µF, calculate the % ripple voltage, % voltage drop, Maximum output voltage and optimum number of stages. | CO3 | (08) |
|  | b) | With neat sketch explain the construction and working of Van-De Graff generator. | CO3 | (08) |
|  | c) | Derive an expression for ripple voltage in a Half wave rectifier. | CO3 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | A 12-stage impulse generator has 0.126 pF condensers. The wave front and the wave tail resistances connected are 800 ohms and 5000 ohms respectively. If the load condenser is 1000 pF, find the front and tail times of the impulse wave produced. | CO4 | (04) |
|  | b) | List all the components used in the Marx generator. | CO4 | (06) |
|  | c) | With a neat sketches discuss different methods used for generation of switching surges. | CO4 | (10) |
| 8. | a) | Draw a current impulse wave and discuss the generation of the same and also explain how inductance is controlled in the circuit | CO4 | (10) |
|  | b) | How are rectangular current pulses generated for testing purposes? How is their time duration controlled? | CO4 | (06) |
|  | c) | Explain the working principle of trigatron gap arrangement. | CO4 | (04) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain how a sphere gap can be used to measure the peak value of voltages. What are the parameters and factors that influence such voltage measurement? | CO5 | (10) |
|  | b) | Show that series resistance microammeter measures high voltage dc voltages. | CO5 | (06) |
|  | c) | A generating voltmeter has to be designed so that it can have a range from 20 to 200 kV dc. If the indicating meter reads a minimum current of 2µA and maximum current of 25µA, what should the capacitance of the generating voltmeter be? | CO5 | (04) |
| 10. | a) | Discuss the principle of potential dividers for the measurement of high voltages. | CO5 | (04) |
|  | b) | Discuss the resistance potential divider for very low impulse voltages and fast rising pulses. | CO5 | (08) |
|  | c) | Discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO5 | (08) |

****************