RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EE73 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain different ionization processes in gases. | CO1 | (12) |
|  | b) | Derive an expression for the current growth in gases considering the secondary ionization process. | CO1 | (08) |
| 2. | a) | What are electronegative gases? Derive the criterion for breakdown in electronegative gases. | CO1 | (08) |
|  | b) | In a parallel wire system placed in air, the radius of wire is 0.1 inch. Assume, smooth surface for the wire, pressure 910 mbar and temperature 30oC. Calculate the corona inception gradient. | CO1 | (06) |
|  | c) | Determine the breakdown voltage for air gaps of 2mm and 5mm lengths under uniform field and standard atmospheric conditions. Also find the voltage if atmospheric pressure is 750mm of Hg and temperature 35oC. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | With neat sketch explain the construction and working of a Van de graff generator for the generation of HVDC. | CO1 | (10) |
|  | b) | Explain the suspended particle theory for the breakdown in a liquid dielectric considering role of viscous force and diffusion. | CO1 | (08) |
|  | c) | What are commercial liquid dielectrics and how are they different from pure liquid dielectrics? | CO1 | (02) |
| 4. | a) | Derive expressions for ripple, voltage drop, optimum number of stages and maximum possible output voltage of Cockroft-Walton cascaded generator. | CO1 | (10) |
|  | b) | With necessary equations and waveforms explain the breakdown in a solid dielectric due to internal discharges. | CO1 | (08) |
|  | c) | List any two actual causes for the bubble formation in liquid dielectrics. | CO1 | (02) |
|  |  | UNIT- III |  |  |
| 5. | a) | With neat sketch explain the generation of high voltage AC using cascade transformers and also list its limitations. | CO3 | (08) |
|  | b) | Draw a Marx generator circuit diagram to generate high voltage DC and explain the working of it. | CO3 | (08) |
|  | c) | Sketch standard lightning and switching impulse wave shapes. | CO3 | (04) |
| 6. | a) | Draw a current impulse wave and discuss the generation of same and also explain how inductance is controlled in the circuit. | CO2 | (08) |
|  | b) | With neat circuit diagram explain the generation of high voltage with high frequency and list its advantages. | CO3 | (08) |
|  | c) | A 12-stage impulse generator has 0.126 pF condensers. The wavefront and the wave tail resistances connected are 800 ohms and 5000 ohms respectively. If the load condenser is 1000 pF, find the front and tail times of the impulse waveproduced. | CO2 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | With a neat circuit arrangement explain the measurement of high voltages using standard spheres. | CO4 | (08) |
|  | b) | Explain the Chubb Fortscue method for measurement of high voltage AC peak. | CO4 | (08) |
|  | c) | List the merits and demerits of High ohmic series resistance with microammeter method for measurement of HVDC. | CO4 | (04) |
| 8. | a) | With neat sketch explain the construction and working of Electrostatic voltmeter for the measurement of high voltages. | CO4 | (10) |
|  | b) | Explain resistance potential divider for measurement of very low impulse voltages and fast rising pulses. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the Schering bridge arrangement for ground capacitors with neat circuit diagram. | CO5 | (08) |
|  | b) | List the tests to be performed on the insulators and discuss any one in detail. | CO5 | (08) |
|  | c) | What is partial discharge? And discuss the significance of it. | CO5 | (04) |
| 10. | a) | List the tests to be carried out on bushings and explain any one in detail. | CO5 | (08) |
|  | b) | With neat circuit diagram explain the straight detection method for the measurement of partial discharge. | CO5 | (08) |
|  | c) | List the tests to be performed on the transformer. | CO5 | (04) |

****************