RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS – MAY/JUNE 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EEPE43 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With reference to the breakdown of gaseous dielectric, define the following terms. - Mobility of Ions - Elastic collisions - Mean free path - Collision of cross section - Self-sustained discharge. | CO1 | (10) |
|  | b) | State and prove Paschen’s law. Discuss the significance of it. | CO1 | (08) |
|  | c) | Write the expression for electron attachment and current growth in Electronegative gases. | CO1 | (02) |
| 2. | a) | What is ionization? List and discuss the causes of primary and secondary ionization process in a gaseous dielectric medium. | CO1 | (10) |
|  | b) | With relevant equations, discuss the formation of avalanche leading to streamer causing the breakdown of gases. | CO1 | (07) |
|  | c) | Mention the limitations of Townsend’s theory for the breakdown of gases. | CO1 | (03) |
|  |  | UNIT- II |  |  |
| 3. | a) | What are commercial liquid dielectrics? List any four properties of Transformer oil. | CO1 | (06) |
|  | b) | An air bubble of thickness 0.5mm is embedded in a solid insulation 1.0cm thick having relative dielectric constant of 3. The voltage applied across the solid is 30kV RMS of 50Hz. Will the internal discharge occurs in the air bubble? If yes, calculate the voltage at which internal discharge occurs if the break down strength of air is 30kV(Peak)/cm. | CO1 | (06) |
|  | c) | With relevant wave forms and equations discuss the breakdown in solid dielectric due to internal discharges. | CO1 | (08) |
| 4. | a) | Explain why the intrinsic breakdown strength of a solid dielectric is high? Discuss the process in detail. | CO1 | (06) |
|  | b) | With relevant equations, discuss how commercial liquid dielectrics breakdown due to suspended particles and also list the factors influencing the breakdown. | CO1 | (08) |
|  | c) | List the causes for formation of bubble in liquid dielectrics. Discuss how the presence of bubble affects the breakdown strength of liquid dielectrics. | CO1 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain half and full wave rectifiers for the generation of HVDC and list the limitations. | CO3 | (12) |
|  | b) | A ten stage Cockroft-Walton circuit has all capacitors of 0.06µF. The secondary voltage of the supply transformer is 100kV at a frequency of 150Hz. If the load current is 1mA, determine (i)voltage regulation (ii)the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | CO3 | (08) |
| 6. | a) | What is a Tesla coil? How is damped high frequency oscillations obtained from a Tesla coil? List the advantages. | CO2 | (10) |
|  | b) | Explain the working principle of resonant transformer and how is it advantageous over the cascade-connected transformers? | CO2 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Draw a neat sketch of standard impulse wave with its wave front and wave tail times and define the following terms: i)Full impulse ii) Wave front time iii) Wave tail time iv)Chopped wave. | CO4 | (08) |
|  | b) | Discuss why Marx generator is needed. Explain the working of it with neat sketch. | CO4 | (08) |
|  | c) | Discuss different methods which can be used for gap breakdown in Marx generator. | CO4 | (04) |
| 8. | a) | Derive an expression for output voltage of a single stage impulse generator. | CO4 | (08) |
|  | b) | Discuss the method of producing switching impulses using transformer. | CO4 | (06) |
|  | c) | How is the circuit inductance controlled and minimized in impulse current generators? | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Discuss the resistance potential divider for very low impulse voltages and fast rising pulses. | CO5 | (08) |
|  | b) | With neat sketch discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO5 | (08) |
|  | c) | List the methods for measuring High voltage DC. | CO5 | (04) |
| 10. | a) | With neat sketch explain the principle and construction of Electrostatic voltmeter. | CO5 | (08) |
|  | b) | Explain with circuit diagram how micro ammeter with high ohmic series resistance can be used to measure high voltage DC.What are its limitations? | CO5 | (08) |
|  | c) | Mention the factors influencing spark over voltage of sphere gaps. | CO5 | (04) |

****************