RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS  JANUARY – FEBRUARY 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | High Voltage Engineering | Max. Marks | : | 100 |
| Course Code | : | EE73 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain the transition of non-self-sustained to self-sustained discharges in the breakdown of a gaseous dielectrics. Discuss the role of γeαd in the breakdown. | CO1 | (07) |
|  | b) | Determine the breakdown voltage of air gap 5mm and 1cm length under uniform field and at STP conditions. Also find the voltage if atmospheric pressure is 710mm of Hg and temperature is 320C. | CO1 | (06) |
|  | c) | State and explain the Paschen’s law and discuss its significance. | CO1 | (07) |
| 2. | a) | With usual notations, derive an expression for the current growth in a gaseous medium considering both primary and secondary ionization coefficients. | CO1 | (08) |
|  | b) | A solid specimen of dielectric has a dielectric constant of 4, and dissipation factor as 0.01 at a frequency of 60 Hz. If it is subjected to an alternating field of 50kV/cm, calculate the heat generated in the specimen due to the dielectric loss. | CO1 | (04) |
|  | c) | With relevant equations explain the formation of avalanche to streamer leading to breakdown of gaseous dielectric. | CO1 | (08) |
|  |  | UNIT – II |  |  |
| 3. | a) | With necessary equations, explain the suspended particle theory. | CO1 | (10) |
|  | b) | Discuss the thermal breakdown in solid dielectrics. | CO1 | (06) |
|  | c) | A 0.04-inch-thick solid specimen of dielectric has a dielectric constant of 4.2, and tanδ=0.001 at a frequency of 50Hz. If it is subjected to an alternating voltage of 5kV, calculate the heat generated in the specimen due to the dielectric loss. | CO1 | (04) |
| 4. | a) | Explain half and full wave rectifiers for the generation of HVDC and list the limitations. | CO2 | (12) |
|  | b) | A ten stage Cockroft-Walton circuit has all capacitors of 0.06µF. The secondary voltage of the supply transformer is 100kV at a frequency of 150Hz. If the load current is 1mA, determine (i)voltage regulation (ii)the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | CO2 | (08) |
|  |  | UNIT – III |  |  |
| 5. | a) | Why a single unit transformer cannot be used for generating higher voltages? | CO2 | (04) |
|  | b) | Explain the principle of Resonant transformer and discuss its advantages. | CO2 | (08) |
|  | c) | Draw a three stage impulse generator and explain the important parts of it. | CO3 | (08) |
| 6. | a) | A 12-stage impulse generator has capacitors each rated at 0.3 μF, 150 kV. The capacitance of the test specimen is 400 pF. Determine the wave front and wave tail resistance to produce at 1.2/50μ sec. impulse wave. | CO3 | (06) |
|  | b) | With neat sketch explain how the inductance of an impulse current generator is reduced. | CO3 | (06) |
|  | c) | With neat sketches explain the different circuits to generate surge impulse wave. | CO3 | (08) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Explain how a sphere gap can be used to measure the peak value of voltages. What are the parameters and factors that influence such voltage measurement? | CO4 | (12) |
|  | b) | Explain the principle and construction of an electrostatic voltmeter for very high voltage measurements. List the advantages and disadvantages. | CO4 | (08) |
| 8. | a) | Discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO4 | (08) |
|  | b) | A coaxial shunt is to be designed to measure an impulse current of 50kA. If the bandwidth of the shunt is to be at least 10 MHz and if the voltage drop across the shunt should not exceed 50V, find the ohmic value of the shunt and its dimensions. Assume the resistivity of the tube material as 300 nΩ-m. | CO4 | (12) |
|  |  | UNIT – V |  |  |
| 9. | a) | Explain the Schering bridge arrangement for ground capacitors with neat circuit diagram. | CO5 | (08) |
|  | b) | List the tests to be performed on the insulators and discuss any one in detail. | CO5 | (08) |
|  | c) | What is partial discharge? Discuss the significance of it. | CO5 | (04) |
| 10. | a) | List the tests to be carried out on bushings. | CO5 | (04) |
|  | b) | With neat circuit diagram explain the straight detection method for the measurement of partial discharge. | CO5 | (08) |
|  | c) | List the tests to be performed on the transformer and discuss in detail any one method. | CO5 | (08) |

****************