RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY/AUGUST 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EEPE43 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  |  | UNIT – I |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) |  | Define Townsend’s first and second ionization coefficients and derive an expression for the current growth in a gas discharge due to secondary ionization. | CO1 | (10) |
|  | b) |  | State and explain Paschen’s law and explain its significance. | CO1 | (07) |
|  | c) |  | Mention the limitations of Townsend’s Criterion. | CO1 | (03) |
| 2. | a) |  | Discuss the time lag in breakdown. | CO1 | (06) |
|  | b) |  | Define the following: i)Mobility ii) Collision due to cross section iii) Mean free path. | CO1 | (06) |
|  | c) |  | Discuss the Streamer theory of breakdown in gases. | CO1 | (08) |
|  |  |  | UNIT – II |  |  |
| 3. | a) |  | What is electromechanical breakdown and derive an expression for electric stress on a solid dielectric during electromechanical breakdown. | CO1 | (10) |
|  | b) |  | List the properties of Transformer oil. | CO1 | (04) |
|  | c) |  | What are commercial liquid dielectrics and how are they different from pure liquid dielectrics? | CO1 | (06) |
| 4. | a) |  | With necessary equations, explain the suspended particle theory. | CO1 | (10) |
|  | b) |  | How does the ‘internal discharge’ phenomenon lead to breakdown in solid dielectrics? | CO1 | (08) |
|  | c) |  | List any two causes for bubble formation in liquid dielectrics. | CO1 | (02) |
|  |  |  | UNIT – III |  |  |
| 5. | a) |  | Derive expressions for ripple, voltage drop, optimum number of stages and maximum possible output voltage of Cockroft-Walton cascaded generator. | CO3 | (12) |
|  | b) |  | Explain half wave rectifier for the generation of HVDC and list the limitations. | CO3 | (08) |
| 6. | a) |  | With neat circuit diagram, explain working of resonant transformer and also mention its advantages and disadvantages. | CO2 | (10) |
|  | b) |  | A ten stage Cockroft-Walton circuit has all capacitors of 0.06µF. The secondary voltage of the supply transformer is 100kV at a frequency of 150Hz. If the load current is 1mA, determine: (i) voltage regulation (ii) the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | CO3 | (08) |
|  | c) |  | Discuss the need of Cascade Transformer. | CO2 | (02) |
|  |  |  | UNIT – IV |  |  |
| 7. | a) |  | What are the parameters mentioned in specification of an Impulse Generator and give an example. | CO4 | (04) |
|  | b) |  | What is the need of multistage impulse generator? And explain its components. | CO4 | (10) |
|  | c) |  | Draw a neat standard impulse wave with its wave front and wave tail times and define the following terms. i) Full impulse ii) Wave front time iii) Wave tail time iv) Chopped wave. | CO4 | (06) |
| 8. | a) |  | Draw different switching impulse circuits and explain any one in detail. | CO4 | (10) |
|  | b) |  | Mention different methods used in triggering of impulse generator and explain with neat sketch Trigatron Gap arrangement. | CO4 | (10) |
|  |  |  | UNIT – V |  |  |
| 9. |  | a) | Explain the sphere gap arrangement for the measurement of high voltage and discuss the factors affecting the measurement. | CO5 | (10) |
|  |  | b) | Explain the Chubb Fortscue method for measurement of high voltage AC peak. | CO5 | (10) |
| 10. |  | a) | Explain with circuit diagram how micro ammeter with high ohmic series resistance can be used to measure high voltage DC and what are its limitations. | CO5 | (08) |
|  |  | b) | Discuss the principle of potential dividers for the measurement of high voltages. | CO5 | (04) |
|  |  | c) | Discuss the resistance potential divider for very low impulse voltages and fast rising pulses. | CO5 | (08) |

****************