RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

EXAMINATIONS SEPTEMBER /OCTOBER 2020

SUPPLEMENTARY SEMESTER / GRADE IMPROVEMENT/ RE –REGISTERED CANDIDATES

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | High Voltage Engineering | Max. Marks | : | 100 |
| Course Code | : | EE73 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the role of secondary parameters in the breakdown of a gaseous medium. | CO1 | (06) |
|  | b) | With relevant equations, discuss the streamer theory for the breakdown of gaseous medium. | CO1 | (08) |
|  | c) | Determine the breakdown voltage of air gap 6mm and 1cm length under uniform field and at STP conditions. Also find the voltage if atmospheric pressure is 720mm of Hg and temperature is 400C. | CO1 | (06) |
| 2. | a) | State and prove paschen’s law. | CO1 | (06) |
|  | b) | Derive an expression for the current growth in gaseous insulating medium considering secondary ionization factor and list the limitations of Townsend’s criteria. | CO1 | (08) |
|  | c) | Write a note on Time lag in the breakdown of a gaseous medium. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Draw a three stage Cockcroft Walton voltage multiplier circuit and derive the expression for voltage drop for the same circuit. | CO2 | (08) |
|  | b) | What is treeing and tracking? Discuss how they influence the breakdown of a solid insulating medium. | CO1 | (06) |
|  | c) | Show that the power output of a Van De Graff generator is P=VI when the losses are ignored | CO2 | (06) |
| 4. | a) | A ten stage Cockcroft-Walton circuit has all capacitors of 0.08μF. The secondary voltage of the supply transformer is 70.7 kV (RMS) at a frequency of 100 Hz. If the load current is 1 mA, determine (i)voltage regulation (ii) the ripple (iii) the optimum number of stages for maximum output voltage (iv)the maximum output voltage. | CO2 | (08) |
|  | b) | Explain the thermal breakdown of a solid dielectric and discuss the condition for thermal breakdown in terms of Electric stress. | CO1 | (06) |
|  | c) | Discuss the role of bubbles in the breakdown of liquid insulating medium. | CO1 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain the working principle of resonant transformer and how is it advantageous over the cascade-connected transformers? | CO2 | (10) |
|  | b) | For the single stage impulse generator shown in the Fig.5(b), obtain an expression for the output. Fig.5(b) | CO3 | (10) |
| 6. | a) | A 5-stage impulse generator has capacitors each rated at 0.05µF, 20kV. The capacitance of the test specimen is 1000pF. Determine the wave front, wave tail resistances and efficiency to produce a 1.2/50µs impulse wave using approximate formulae. | CO3 | (06) |
|  | b) | How are rectangular current pulses generated for testing purposes? How is their time duration controlled? | CO3 | (06) |
|  | c) | An impulse current generator is rated for 90kW-s. The parameters of the circuit are C=53µF, L=1.47µH and the dynamic resistance=0.0156 ohm. Determine the time-to-front and the time-to-tail of the current waveform. | CO3 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain how a sphere gap can be used to measure the peak value of voltages. What are the parameters and factors that influence such voltage measurement? | CO4 | (12) |
|  | b) | Explain the principle and construction of an electrostatic voltmeter for very high voltage measurements. List the advantages and disadvantages. | CO4 | (08) |
| 8. | a) | Discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO4 | (08) |
|  | b) | Explain the resistance divider for impulse voltage measurement with impedance matching at the divider end. | CO4 | (12) |
|  |  | UNIT- V |  |  |
| 9. | a) | With neat circuit diagram explain the balance detection method for the measurement of partial discharge. | CO5 | (08) |
|  | b) | Explain the Schering bridge arrangement for ground capacitors with neat circuit diagram. | CO5 | (08) |
|  | c) | List the tests to be performed on the transformer. | CO5 | (04) |
| 10. | a) | List the tests to be carried out on bushings and explain any one in detail. | CO5 | (08) |
|  | b) | List the tests to be performed on the insulators and discuss any two methods in detail. | CO5 | (10) |
|  | c) | What is partial discharge? | CO5 | (02) |

***************