| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)

Accredited by NBA & NAAC with ‘A+’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – SEPTEMBER 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | High Voltage Engineering | Max. Marks | : | 100 |
| Course Code | : | EE73 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer any five full questions.

| 1. | a) |  | Explain different ionization processes in gases. | CO1 | (12) |
| --- | --- | --- | --- | --- | --- |
|  | b) |  | Derive an expression for the current growth in gases considering the secondary ionization process. | CO1 | (08) |
| 2. | a) |  | What is time lag in breakdown? Explain how this is useful in designing the protective scheme. | CO1 | (08) |
|  | b) |  | In a parallel wire system placed in air, the radius of wire is 0.1 inch. Assume, smooth surface for the wire, pressure 910 mbar and temperature 30oC. Calculate the corona inception gradient. | CO1 | (06) |
|  | c) |  | Determine the breakdown voltage for air gaps of 2mm and 5mm lengths under uniform field and standard atmospheric conditions. Also find the voltage if atmospheric pressure is 750mm of Hg and temperature 35oC. | CO1 | (06) |
| 3. | a) |  | With necessary equations, explain the suspended particle theory. | CO1 | (10) |
|  | b) |  | Discuss the thermal breakdown in solid dielectrics. | CO1 | (06) |
|  | c) |  | A 0.1cm thick solid specimen of dielectric has a dielectric constant of 4.2, and tanδ=0.001 at a frequency of 50Hz. If it is subjected to an alternating voltage of 5kV, calculate the heat generated in the specimen due to the dielectric loss. | CO1 | (04) |
| 4. | a) |  | Explain half and full wave rectifiers for the generation of HVDC and list the limitations. | CO2 | (12) |
|  | b) |  | A ten stage Cockroft-Walton circuit has all capacitors of 0.06µF. The secondary voltage of the supply transformer is 100kV at a frequency of 150Hz. If the load current is 1mA, determine (i)voltage regulation (ii) the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | CO2 | (08) |
| 5. | a) |  | Explain the working principle of resonant transformer and how is it advantageous over the cascade-connected transformers? | CO2 | (10) |
|  | b) |  | For the single stage impulse generator shown in the Fig.5(b), obtain an expression for the output. Fig.5(b) | CO3 | (10) |
| 6. | a) |  | A 5-stage impulse generator has capacitors each rated at 0.05µF, 20kV. The capacitance of the test specimen is 1000pF. Determine the wave front, wave tail resistances and efficiency to produce a 1.2/50µs impulse wave using approximate formulae. | CO3 | (06) |
|  | b) |  | How are rectangular current pulses generated for testing purposes? How is their time duration controlled? | CO3 | (06) |
|  | c) |  | An impulse current generator is rated for 90kW-s. The parameters of the circuit are C=53µF, L=1.47µH and the dynamic resistance=0.0156 ohm. Determine the time-to-front and the time-to-tail of the current waveform. | CO3 | (08) |
| 7. | a) |  | Explain how a sphere gap can be used to measure the peak value of voltages. What are the parameters and factors that influence such voltage measurement? | CO4 | (12) |
|  | b) |  | Explain the principle and construction of an electrostatic voltmeter for very high voltage measurements. List the advantages and disadvantages. | CO4 | (08) |
| 8. | a) |  | Discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO4 | (08) |
|  | b) |  | Explain the resistance divider for impulse voltage measurement with impedance matching at the divider end. | CO4 | (12) |
| 9. |  | a) | Explain the high voltage Schering bridge for the tanδ and capacitance measurement of insulation. | CO5 | (10) |
|  |  | b) | What are partial discharge? Discuss the method of balanced detection for locating partial discharges in electrical equipment. | CO5 | (10) |
| 10. |  | a) | Explain different types of tests conducted on Transformers. | CO5 | (12) |
|  |  | b) | Discuss the types of tests conducted on high voltage bushings. | CO5 | (08) |

****************