RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SUPPLEMENTARY SEMESTER EXAMINATIONS –  AUGUST 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EEPE43 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Define Townsend’s first and second ionization coefficients. How is the condition for breakdown obtained in a Townsend discharge? | CO1 | (08) |
|  | b) | Discuss the secondary ionization processes. | CO1 | (08) |
|  | c) | In an experiment in a certain gas it was found that the steady state current is 5.5x10-8A at 8kV at a distance of 0.4cm between the plane electrodes. Keeping the field constant and reducing the distance to 0.1cm results in a current of 5.5x10-9A. Calculate Townsend’s primary ionization coefficient. | CO1 | (04) |
| 2. | a) | Discuss the Streamer theory of breakdown in gases. | CO1 | (08) |
|  | b) | In a parallel wire system placed in air, the radius of wire is 0.1 inch. Assume, smooth surface for the wire, pressure 910 mbar and temperature 30oC. Calculate the corona inception gradient. | CO1 | (06) |
|  | c) | Discuss the time lag in breakdown. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | What is the condition for thermal breakdown of a solid dielectric? Explain the process with necessary equations. | CO1 | (06) |
|  | b) | An AC voltage of 84.85kV (peak) is applied between two parallel plates rounded at the edges and placed 2.5cm apart in air. A press board sheet of thickness 0.25cm is placed on the lower plate, in parallel with two plates. Calculate how 84.85kV (Peak) is distributed across press board sheet and air. Dielectric constant of press board sheet is 4. | CO1 | (06) |
|  | c) | Prove that solid dielectrics will breakdown when the material gets compressed more than 40% of its original thickness due to electrical stress. | CO1 | (08) |
| 4. | a) | What is treeing and tracking of a solid dielectric? Discuss the breakdown of solid dielectric due to the same with a sketch. | CO1 | (08) |
|  | b) | With relevant wave forms and equations discuss the breakdown in solid dielectric due to internal discharges | CO1 | (08) |
|  | c) | Discuss the stressed oil volume theory for the breakdown in liquid dielectrics. | CO1 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | Mention the advantages of High Frequency Transformers. | CO2 | (04) |
|  | b) | Explain Greinechar voltage doubler circuit for generating High Voltage DC. | CO3 | (06) |
|  | c) | Derive expressions for ripple, voltage drop, optimum number of stages and maximum possible output voltage of Cockcroft-Walton voltage multiplier circuits. | CO3 | (10) |
| 6. | a) | A ten stage Cockroft-Walton circuit has all capacitors of 0.08µF. The secondary voltage of the supply transformer is 150kV at a frequency of 100Hz. If the load current is 1mA, determine (i)voltage regulation (ii)the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | CO3 | (08) |
|  | b) | Explain the working principle of resonant transformer and how is it advantageous over the cascade-connected transformers? | CO2 | (08) |
|  | c) | List the limitations of Half wave rectifier. | CO3 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Define standard Lightning and Switching impulse voltages. | CO4 | (04) |
|  | b) | Discuss the methods of producing switching impulses. | CO4 | (10) |
|  | c) | A 8-stage impulse generator has capacitors each rated at 0.08µF, 20kV. The capacitance of the test specimen is 1000pF. Determine the wave front, wave tail resistances and efficiency to produce a 1.2/50µs impulse wave using approximate formulae. | CO4 | (06) |
| 8. | a) | Define standard impulse current wave. | CO4 | (02) |
|  | b) | How are rectangular current pulses generated for testing purposes? How is their time duration controlled? | CO4 | (06) |
|  | c) | An impulse current generator is rated for 60kW-s. The parameters of the circuit are C=53µF, L=1.47µH and the dynamic resistance=0.0156 ohm. Determine the peak value of the current and the time-to-front and the time-to-tail of the current waveform. | CO4 | (06) |
|  | d) | What is a trigatron gap? Explain it functions and operation. | CO4 | (06) |
|  |  | UNIT - V |  |  |
| 9. | a) | With neat sketch explain the working and construction of Generating Voltmeter for the measurement of HVDC. | CO5 | (08) |
|  | b) | What is impedance matching? How it can be achieved? | CO5 | (04) |
|  | c) | Explain the factors affecting the measurement of high voltages using standard sphere gap method. | CO5 | (08) |
| 10. | a) | Discuss the merits and demerits of Generating voltmeter for the measurement of HVDC. | CO5 | (04) |
|  | b) | With neat arrangement sketch explain the measurement of high voltages using standard sphere gap method. | CO5 | (08) |
|  | c) | With relevant waveforms and sketch explain resistance potential divider for very low impulse voltage and fast rising pulses. | CO5 | (08) |

****************