RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  JULY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | High Voltage Engineering | Max. Marks | : | 100 |
| Subject Code | : | EE73 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | List the factors affecting primary and secondary ionization and discuss their influence on the breakdown of gaseous dielectric. | CO1 | (10) |
|  | b) | Discuss breakdown in electronegative gases. | CO1 | (06) |
|  | c) | In an experiment in a certain gas it was found that the steady state current is 5.5x10-8A at 8kV at a distance of 0.4cm between the plane electrodes. Keeping the field constant and reducing the distance to 0.1cm results in a current of 5.5x10-9A. Calculate Townsend’s primary ionization coefficient. | CO1 | (04) |
| 2. | a) | Define Townsend’s first and second ionization coefficients and derive an expression for the current growth in a gas discharge due to secondary ionization. | CO1 | (10) |
|  | b) | Define the following: i) Elastic collisions ii) Inelastic collisions iii) Mean free path. | CO1 | (06) |
|  | c) | List any four important properties of a gaseous dielectric. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | With necessary equations, explain the suspended particle theory. | CO1 | (10) |
|  | b) | Discuss the thermal breakdown in solid dielectrics. | CO1 | (06) |
|  | c) | A 0.1cm thick solid specimen of dielectric has a dielectric constant of 4.2, and tanδ=0.001 at a frequency of 50Hz. If it is subjected to an alternating voltage of 5kV, calculate the heat generated in the specimen due to the dielectric loss. | CO1 | (04) |
| 4. | a) | Explain half and full wave rectifiers for the generation of HVDC and list the limitations. | CO2 | (12) |
|  | b) | A ten stage Cockroft-Walton circuit has all capacitors of 0.06µF. The secondary voltage of the supply transformer is 100kV at a frequency of 150Hz. If the load current is 1mA, determine (i)voltage regulation (ii)the ripple (iii) the optimum number of stages for maximum output voltage (iv) the maximum output voltage. | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | With neat circuit diagram, explain working of resonant transformer and also mention its advantages and disadvantages. | CO3 | (10) |
|  | b) | Derive an expression for the output voltage of a single stage impulse generator. | CO3 | (08) |
|  | c) | Mention the standard switching and lightning impulse durations. | CO3 | (02) |
| 6. | a) | Draw different switching impulse circuits and explain any one in detail. | CO2 | (08) |
|  | b) | Describe the circuit arrangement for producing high lightning current waveforms in the laboratories. | CO3 | (08) |
|  | c) | A 12-stage impulse generator has capacitors each rated at 0.3µF, 150kV. The capacitance of the test specimen is 400 pF. Determine the wave front and wave tail resistances to produce a 1.2/50µs impulse wave using approximate formulae. | CO2 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain how a sphere gap can be used to measure the peak value of voltages. What are the parameters and factors that influence such voltage measurement? | CO4 | (12) |
|  | b) | Explain the principle and construction of an electrostatic voltmeter for very high voltage measurements. List the advantages and disadvantages. | CO4 | (08) |
| 8. | a) | Discuss the Chubb-Fortescue method for ac peak voltage measurement. | CO4 | (08) |
|  | b) | Explain the resistance divider for impulse voltage measurement with impedance matching at the divider end. | CO4 | (12) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the Mole bridge arrangement for the low frequency measurement. | CO5 | (08) |
|  | b) | List the tests to be performed on the insulators and discuss any one in detail. | CO5 | (08) |
|  | c) | List the tests to be performed on the transformer. | CO5 | (04) |
| 10. | a) | List the tests to be carried out on bushings and explain any one in detail. | CO5 | (08) |
|  | b) | With neat circuit diagram explain the balance detection method for the measurement of partial discharge. | CO5 | (08) |
|  | c) | What is partial discharge? Discuss the significance of it. | CO5 | (04) |

****************