RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Power Systems - II | Max. Marks | : | 100 |
| Subject Code | : | EE72 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Derive the admittance and impedance form of the primitive element. | CO1 | (05) |
|  | b) | The partial Zbus of a network is given by: pu The three nodes are 1,2,and 3 respectively. Now an element of impedance j0.6pu is added between nodes 1 and 2. What is the new Zbus? | CO1 | (07) |
|  | c) | The line data of a four bus system is as follows: Line; Impedance(pu). 1-2; 0.05+j0.15. 1-3; 0.1+j0.3. 2-3; 0.15+j0.45. 2-4; 0.1+j0.3. 3-4; 0.05+j0.15. Form the bus admittance matrix. If a shunt capacitance of admittance j0.12 pu is connected at bus 3, how will the Ybus be modified? | CO1 | (08) |
| 2. | a) | Derive the π model for a transformer connected between bus p –q, with an off-nominal ratio a. A transformer is connected between buses 4-3 in a network and has an impedance of j0.133pu, with an off-nominal ratio of 0.909. Obtain the  model . | CO1 | (05) |
|  | b) | The reduced branch-node incidence matrix of a network is given by Ar (bxn)= Draw the oriented graph. Draw a tree, corresponding co-tree, Basic cut-set incidence matrix and Basic loop incidence matrix. What are the describing equations for the two matrices? ? | CO1 | (06) |
|  | c) | In a system with 20 nodes, 45 elements, what is the size of the incidence matrix, number of tree branches, number of links, basic cutset matrix and basic loop incidence matrix. | CO1 | (04) |
|  | d) | Derive the equations to modify the elements of a partial Zbus matrix, when a link is added between buses p-q. | CO1 | (05) |
|  |  | UNIT - II |  |  |
| 3. | a) | What is the significance of load flow analysis in a power system? What are the different types of buses as categorized for load flow studies, explain briefly each one of them. | CO2 | (08) |
|  | b) | Obtain the load flow solution at the end of first iteration using Gauss Seidel method for the network shown in fig. 3(b). SB; EB; R; X. 1; 2; 0.05; 0.15. 1; 3; 0.1; 0.3. 2; 3; 0.15; 0.45. 2; 4; 0.1; 0.3. 3; 4; 0.05; 0.15. Fig. 3(b) Bus No; P (pu); Q (pu); V (pu). 1; -; -; 1.04 0. 2; 0.5; -0.2; -. 3; -1.0; 0.5; -. 4; -0.3; -0.1; -. | CO2 | (08) |
|  | c) | Compare Gauss-Seidel and Newton-Raphson methods for load flow analysis. | CO2 | (04) |
| 4. | a) | Obtain the voltages at all buses for the three bus system shown in fig. 4(a) at the end of first iteration by NR method. Fig 4 (a) Line Data StartBus; End Bus; R(pu); X(pu); Bc/2. 1; 2; 0.0; 0.1; 0.0. 1; 3; 0.0; 0.2; 0.0. 2; 3; 0.0; 0.2; 0.0. Bus Data Bus no.; Pg; Qg; Pl; Ql; Vsp. 1(slack); -; -; -; -; 1.0. 2(PV); 5.3217; -; -; -; 1.1. 3(PQ); -; -; 3.6392; 0.5339; -. | CO2 | (14) |
|  | b) | List the data essential for the load flow? | CO2 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | With necessary assumptions, derive the transmission loss formula for a system consisting of n-generating plant supplying several loads interconnected through a transmission network. | CO3 | (10) |
|  | b) | Determine the economic operation for the three thermal units when delivering a total of 1200MW. Unit A: Pmax= 600MW, Pmin =150MW FA=500+7PGA + 0.0015 PGA2 Unit B: Pmax= 500MW, Pmin =125MW FB=300+7.88PGB + 0.002 PGB2 Unit C: Pmax= 300MW, Pmin =75MW FC=80+7.99PGC + 0.005 PGC2 | CO3 | (06) |
|  | c) | Define and draw the heat rate curve and incremental fuel rate curve. | CO3 | (04) |
| 6. | a) | Determine the condition for the economic scheduling of generators without limits and without losses. | CO3 | (04) |
|  | b) | The fuel cost functions in $/h for three thermal plants are given by F1=350+7.2PG1 + 0.004 PG12 F2=500+7.3PG2 + 0.025 PG22 F3=600+6.74PG3+ 0.003 PG32 PG1, PG2 and PG3 are in MW. Find the optimal schedule and compare the cost of this to the case when the generators share load equally if i) PD=500MW ii) PD=1000MW | CO3 | (08) |
|  | c) | Derive an expression for the incremental lambda with losses using iterative technique. | CO3 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain the solution of swing equations by point by point method. | CO4 | (10) |
|  | b) | For the swing equation, 0.0331d2δdt2 = 0.8−1.333 sin δ Calculate δ and ω after 0.05sec by taking a step by 0.05sec. Given δ0=0.756, ω0=2.067. Use Runge-Kutta method. | CO4 | (10) |
| 8. |  | A 50Hz, 500MVA, 4ookV generator (with transformer) is connected to a 400kV infinite bus bar through an interconnector. The generator has H=2.5 MJ/MVA, voltage behind transient reactance is 450kV and is loaded 460 MW. The transfer reactance between generator and bus bar under various conditions are: Pre Fault:0.5 pu During fault:1.0 pu Post Fault: 0.75 pu Calculate the swing curve using intervals of 0.05 sec and assuming that the fault is cleared at 7.5 cycles. | CO4 | (20) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the objective and specification of load compensation. | CO5 | (10) |
|  | b) | With a block diagram, describe the single area Automatic load Frequency control (ALFC). | CO5 | (05) |
|  | c) | Explain the operation of static VAR Compensator (SVC). | CO5 | (05) |
| 10. | a) | With neat figure , explain speed governing system of a steam turbine | CO5 | (10) |
|  | b) | Discuss the factors which limits the loading capacity of transmission lines | CO5 | (05) |
|  | c) | What is SSSC? Describe its operation. | CO5 | (05) |

****************