RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Power Systems - II | Max. Marks | : | 100 |
| Course Code | : | EE72 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Suitably assume any missing data.

|  |  |  | UNIT – I |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) |  | Explain the terms with example: i) Graph ii) Tree. | CO1 | (04) |
|  | b) |  | Derive an expression for formation of Bus admittance matrix by singular transformation method. | CO1 | (06) |
|  | c) |  | For the system shown in Fig.1(c) find Z bus matrix by building algorithm. Fig. 1(c) | CO1 | (10) |
| 2. | a) |  | Explain primitive network in impedance form and admittance form. | CO1 | (06) |
|  | b) |  | The bus incidence matrix A for the networks of 8 elements and 5 nodes is given below. Reconstruct the oriented graph. Hence obtain the one-line diagram of the system indicating the generator positions. | CO1 | (06) |
|  | c) |  | Derive the expression for Z bus matrix of a system when a link is added to an existing system. | CO1 | (08) |
|  |  |  | UNIT – II |  |  |
| 3. | a) |  | With relevant equations for line flows explain the method of calculating system losses and slack bus generation from load flow solution. | CO2 | (08) |
|  | b) |  | For the system shown in Fig.3(a) below obtain the voltages at all buses at the end of first iteration using GS technique. 0.02+ j0.08 0.02+ j0.08 0.02+ j0.08 1 3 2 Fig. 3(a) Bus; GEN; LOAD; V in pu; Bus type. Pg; Qg; Pd; Qd. 1; -; -; -; -; 1.05+j0; Slack. 2; 3; -; -; -; 1.0; PV. 3; -; -; 4; 2; -; PQ. | CO2 | (12) |
| 4. | a) |  | Deduce fast decoupled load flow model. Clearly state all the assumptions made. | CO2 | (06) |
|  | b) |  | Explain the algorithm of Newton Raphson load flow analysis in polar coordinates. | CO2 | (06) |
|  | c) |  | For the system shown in Fig. 4(c) find the voltage at the end of first iteration using NRLF method. The load is 1+j0.5 pu and the slack bus voltage is 1+j0 pu. Line impedance is 0.12+j0.16 pu. LOAD Slack Bus Fig. 4(c) | CO2 | (08) |
|  |  |  | UNIT – III |  |  |
| 5. | a) |  | Deduce the condition for optimal load dispatch neglecting transmission losses in a system. . | CO3 | (04) |
|  | b) |  | Consider the following incremental cost curves in Rs/ MWhr for a plant having 2 units: Calculate the extra cost incurred in Rs/hr if a load of 220MW is scheduled as MW, compared to optimal scheduling neglecting losses. | CO3 | (06) |
|  | c) |  | Derive the expression for transmission loss coefficients. Clearly state the assumptions made. | CO3 | (10) |
| 6. | a) |  | Fig.6(a) shows the system having two plants. The reference bus voltage is 1+ j0 pu. Calculate the loss coefficients of the system in pu and reciprocal of MW if the base MVA is 100. Ia=2-j0.5 pu Ib=1-j0.25 pu Ic=1.6-j0.4 pu Za=0.015+j0.06 pu Zb=0.015+j0.06 pu Zc=0.01+j0.04 pu G2 Ref Ia Ic Ib G1 L1 L2 Fig. 6(a) | CO3 | (12) |
|  | b) |  | A two bus system is shown in Fig.6.(b) If 100 MW is transmitted from plant1to the load a transmission loss of 10 MW is incurred. find the generation by from each of the load and the power received by the load when system λ=25 Rs/MWhr Fig.6(b) Plant1 Plant 2 LOAD | CO3 | (08) |
|  |  |  | UNIT – IV |  |  |
| 7. | a) |  | Explain step by step method for solving swing equation. State the assumptions made. | CO4 | (10) |
|  | b) |  | Explain the modified Euler’s method used in the solution of swing equation under transient stability studies. | CO4 | (10) |
| 8. | a) |  | Illustrate clearly the steps involved in solving swing equation using Runge- Kutta method for transient analysis. | CO4 | (10) |
|  | b) |  | At the beginning of a particular iteration in solution of swing equation, and rad/ sec. The mechanical power input =0.8 p.u. and the inertial constant M=0.0331 s2/ rad. If the maximum electrical power is 1.333p.u., determine the values of and , for the next 3 iterations using point by point method, choosing step size of 0.05 sec. | CO4 | (10) |
|  |  |  | UNIT – V |  |  |
| 9. |  | a) | Two generators rated 200MW and 400MW are operating in parallel. The drop characteristics of their governors are 4% and 5% respectively from no-load to full load. Assuming that the generators are operating at 50Hz at no load, how would a load of 400MW be shared between them? What will be the system frequency at this load? Assume free governor operation. | CO5 | (06) |
|  |  | b) | Write a brief note on TCSC controller. | CO5 | (06) |
|  |  | c) | Obtain the block diagram model of generator load model for ALFC. | CO5 | (08) |
| 10. |  | a) | Starting from block diagram model of LFC of isolated power system, derive the expressions for steady state deviation in frequency. | CO5 | (10) |
|  |  | b) | Write short notes on AVR. | CO5 | (05) |
|  |  | c) | Compare series and shunt compensation of power system. | CO5 | (05) |

****************