RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKE UP EXAMINATIONS –  APRIL 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Power Systems - II | Max. Marks | : | 100 |
| Course Code | : | EE72 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Suitably assume any missing data.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With neat sketch Model the 𝜋-equivalent circuit of transformers with off-nominal turns ratio used in formation of bus admittance matrix. | CO1 | (08) |
|  | b) | What are primitive element and primitive network? Mention an example for each. | CO1 | (04) |
|  | c) | For the network shown in Fig. 1(c) obtain the bus admittance matrix YBUS by singular transformation analysis. The line data is as shown in the table below. Line No; Connecting nodes; Admittance (pu). 1; 1-4; 1.4. 2; 1-2; 1.6. 3; 2-3; 2.4. 4; 3-4; 2.0. 5; 2-4; 1.8. Fig. 1(c) | CO1 | (08) |
| 2. | a) | Fig. 2(a) Consider the power system shown in Fig. 2(a) and corresponding data is given in Table 2.a. A static shunt capacitor is connected at bus 4 with admittance j0.005pu. Formulate the Y-Bus using inspection method. Table 2.a Element; From-To Bus; Line impedance; Half line charging admittance; Off-nominal turn ratio.. 1; 1-4; 0.08+j0.37; 0.007; --. 2; 1-6; 0.123+j0.518; 0.010; --. 3; 2-3; 0.723+j1.05; 0.0; --. 4; 2-5; 0.282+j0.64; 0.0; --. 5; 4-3; 0.0+j0.133; 0.0; 0.909. 6; 4-6; 0.097+j0.407; 0.0076; --. 7; 6-5; 0.0+j0.3; 0.0; 0.976. | CO1 | (10) |
|  | b) | Derive the step-by-step algorithm for formation of bus impedance matrix, when an element is added to the partial network is branch. | CO1 | (10) |
|  |  | UNIT – II |  |  |
| 3. | a) | Obtain the expressions of jacobian of Newton Raphson’s power flow model. | CO2 | (10) |
|  | b) | What is the significance of power flow analysis in a power system? Discuss the classification of various types of buses in a power system for load flow studies. | CO2 | (06) |
|  | c) | Compare gauss Seidel and Newton raphson methods for power flow analysis. | CO2 | (04) |
| 4. | a) | Obtain the power flow solution at the end of first iteration using Gauss Seidel method for the network shown in Fig.4(a). SB; EB; R; X. 1; 2; 0.05; 0.15. 1; 3; 0.1; 0.3. 2; 3; 0.15; 0.45. 2; 4; 0.1; 0.3. 3; 4; 0.05; 0.15. Fig.4(a) Bus No; P (pu); Q (pu); V (pu). 1; -; -; 1.04∠0. 2; 0.5; -0.2; -. 3; -1.0; 0.5; -. 4; -0.3; -0.1; -. | CO2 | (10) |
|  | b) | List the data essential for the power flow? | CO2 | (06) |
|  | c) | What is slack bus? Explain the significance of it in the power flow analysis. | CO2 | (04) |
|  |  | UNIT – III |  |  |
| 5. | a) | In a system with 2 plants, the incremental fuel costs are given by: IC1 = 0.01PG1 + 20 Rs/ MWh IC2 = 0.015PG2 + 22.5 Rs/ MWh The system is running under optimal scheduling with PG1= PG2=100MW. If incremental transmission loss of generator 2 is 0.2, find the penality factor and incremental transmission loss of generator 1. | CO3 | (06) |
|  | b) | Distinguish between the problem of unit commitment and economic dispatch. | CO3 | (04) |
|  | c) | Derive the transmission loss formula in terms of B-coefficients. | CO3 | (10) |
| 6. | a) | The fuel cost functions in Rs./h for 3 thermal plants are given by: F1 = 350 +7.2 PG1 +0.004 P2G1 F2 = 500 +7.3 PG2+0.0025 P2G2 F3 = 600 +6.74 PG3+0.003 P2G3 PG1, PG2 and PG3 are in MW. Find the optimal schedule and compare the cost of these, when the generators share the load equally. Given PD=450MW. | CO3 | (10) |
|  | b) | Compute the loss coefficients for the network shown in the Fig. 6(b), using the following data: Ia= 1.0-j0.15PU, Ib = 0.5-j0.1PU, Ic = 0.2-j0.05PU Za = 0.02-j0.15PU, Zb = 0.03-j0.15PU, Zc = 0.02-j0.25PU Fig. 6(b) | CO3 | (10) |
|  |  | UNIT – IV |  |  |
| 7. | a) | With help of a flow chart, explain the method of finding transient stability of a given power system using modified Euler’s method. | CO4 | (10) |
|  | b) | Explain the solution of swing equation by point-by-point method. Mention the assumption made. | CO4 | (10) |
| 8. | a) | With the of flow chart explain how transient stability studies are carried out using Runge-Kutta method. | CO4 | (10) |
|  | b) | Explain clearly the representation of synchronous machine and load for transient stability studies. | CO4 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | What is load frequency control? Explain with a block diagram, the model of a load frequency control for an isolated power system. | CO5 | (10) |
|  | b) | Write a brief note on: (i) Series Compensation and (ii)FACTS Controllers. | CO5 | (10) |
| 10. | a) | Explain the objectives and functions of ALFC and AVR loops in a power system with a neat block diagram. | CO5 | (10) |
|  | b) | Write a brief note on: (i) Load Compensation and (ii) Line Compensation. | CO5 | (10) |

****************