RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  AUGUST 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Signal Processing | Max. Marks | : | 100 |
| Subject Code | : | EE51 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the need of frequency domain sampling in Digital Signal Processing. | CO1 | (04) |
|  | b) | Compute 4 point DFT of x(n)=cos(n. | CO3 | (06) |
|  | c) | Given two sequences x1(n) = (2, 1, 2, 1), x2(n) = (1, 2, 3, 4). Find x3(n) which is the circular convolution of the above two sequences. Compare the result with linear convolution. | CO2 | (10) |
| 2. | a) | Consider the sequence x(n)= δ (n) + 2δ (n-2)+ δ(n-3). Find 4 point DFT of x(n). If y(n) is the four point circular convolution of x(n) with itself, find y(n) and 4 point DFT Y(K). | CO2 | (08) |
|  | b) | The first five points of the 8 point DFT of a real valued sequence are {28, -j4.966, 4+4j, -4+1.66j, -4}. Determine remaining three points of the DFT. | CO2 | (04) |
|  | c) | Given two sequences x1(n) = (1, 2, -1, -1), x2(n) = (-3, -2, -1, 0). Find x3 (n) which is the circular convolution of the above two sequences using DFT-IDFT method. Verify the answer by matrix method. | CO2 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | The long sequence x(n)= {1,4,3,0,7,4,-7,-7,-1,3,4,3} is filtered through a filter whose impulse response is h(n)= {1, 2}. Compute the output of the filter y(n) using overlap add method. Use 5 point circular convolution, | CO2 | (10) |
|  | b) | Compute the 8-point DFT of the sequence x(n)= {0.5, 0.5, 0.5, 0.5, 0, 0, 0, 0} using radix 2 DIT FFT algorithm. Show the stage wise calculation. | CO3 | (10) |
| 4. | a) | Develop an 8 point DIT FFT algorithm. Draw the signal flow graph. Determine the DFT of the sequence x(n)= {1, 1, 1, 1, 0, 0, 0, 0} using the signal flow graph. Show all intermediate results on the graph. | CO3 | (16) |
|  | b) | Draw the signal flow graph to find the 4 point IDFT using radix 2 DFT-FFT algorithm. | CO2 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | An LTI system is described by the following input-output relation. 2y (n) - y (n-2) +3y (n-3) = 4x (n-2) Realize the system in the following forms: - Direct form I realization. - Direct form II realization. | CO3 | (10) |
|  | b) | Realize the following system in: i) Direct form ii) Cascade form H(z)=1+ ¾ z -1 + 1⅞ z -2 + ¾ z -3 + z -4, | CO2 | (10) |
| 6. | a) | An LTI system is shown in figure below. Write the input- output relationship and Realize the system in the following forms: - Direct form I realization. - Direct form II realization. | CO3 | (12) |
|  | b) | Realize a Linear phase FIR filter with the following impulse response. h(n)= δ(n) + ½ δ(n-1) - ¼ δ(n-2) + δ(n-4)+ ½ δ(n-3). Give necessary equations. | CO2 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Design an FIR low pass digital filter with rectangular window of pass band gain 0dB, cut-off frequency 200Hz and sampling frequency 1kHz. Assume the length of the impulse response as 7. | CO4 | (10) |
|  | b) | The frequency response of a filter is given by: Hd(ejω)=jω for - π≤ ¦ω¦≤ π. Design the filter using rectangular window function. Take N=7. | CO4 | (10) |
| 8. | a) | Explain the linear phase FIR filter design using frequency sampling method. | CO4 | (08) |
|  | b) | Determine the frequency response of the FIR filter if Hamming window is used with N=7 for the following desired frequency response. Hd()= e-j3ω for w ≤3π4 0 for3π4<w < π | CO4 | (12) |
|  |  | UNIT- V |  |  |
| 9. | a) | Design a digital filter using chebyshev approximation and bilinear transformation to meet the following specifications: Passband ripple=1dB for 0≤w≤0.15π Stopband attenuation ≥20dB for 0.45π ≤w≤π | CO4 | (10) |
|  | b) | Describe the architecture of C2xx DSP CPU. | CO5 | (10) |
| 10. | a) | Write a note on TMS320LF2407 digital signal controller. | CO5 | (06) |
|  | b) | Design a Band pass analog filter with the following specifications. i. - 3.013dB lower and upper cut off frequencies of 50 Hz and 20kHz. ii. Stop band attenuation of at least 20 dB at 20 Hz and 45 kHz, iii. Monotonic frequency response. | CO4 | (14) |

****************