RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY 2017

| Course & Branch | : | Electrical & Electronics Engineering. | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Power Systems - II | Max. Marks | : | 100 |
| Subject Code | : | EE702 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  |  | UNIT – I |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) |  | Mention the advantages and application of bus admittance matrix. | CO1 | (04) |
|  | b) |  | Model the 𝜋-equivalent circuit of transformers with off-nominal turns ratio used in formation of bus admittance matrix. | CO1 | (06) |
|  | c) |  | Derive the step-by-step algorithm for formation of bus impedance matrix, when added element to the partial network is branch. | CO1 | (10) |
| 2. | a) |  | Define tree and co-tree with an example. | CO1 | (04) |
|  | b) |  | Derive the standard expression (Ybus=At Ypri A) using singular transformation method. | CO1 | (06) |
|  | c) |  | The one-line diagram of a four bus power system is as shown in Figure (2c). Reactances are given in per unit on a common MVA base. Transformers T1 and T2 have tap settings of 0.8:1 and 1.25:1 respectively. Obtain the bus admittance matrix. | CO1 | (10) |
|  |  |  | UNIT – II |  |  |
| 3. | a) |  | Mention the assumptions made in FDLF load flow analysis. | CO2 | (04) |
|  | b) |  | What are the different types of buses as categorized for load flow studies, explain briefly each one of them. | CO2 | (06) |
|  | c) |  | In the power system network shown in Figure (3c), bus 1 is a slack bus with V1= 1.0∠0° per unit and bus 2 is a load bus with S2= (280+j60) MVA on a base of 100MVA. The line impedance is Z12 = 0.02+j0.04 per unit. Using Gauss-Seidel method, determine V2 at the end of first iteration. Assume an initial estimate of Per unit. | CO2 | (10) |
| 4. | a) |  | Define power flow problem and explain its importance. | CO2 | (04) |
|  | b) |  | Compare the performance of the following load flow methods (i) Gauss Siedel (ii) Newton Raphson (iii) Fast Decoupled. | CO2 | (06) |
|  | c) |  | In the two bus system shown in Figure (4c). The bus 1 is a slack bus with V=1∠0° pu and bus 2 is a load bus with P=100MW and Q=50MVAr on a base of 100MVA. The line impedance is Z12 = (0.12+j0.16) PU. Using Newton-Raphson method, obtain the voltage magnitude and phase angle of bus 2 at the end of first iteration. Assume and . | CO2 | (10) |
|  |  |  | UNIT – III |  |  |
| 5. | a) |  | Derive the transmission loss formula for a system consisting of n-generating plant supplying several loads interconnected through a transmission network. | CO3 | (10) |
|  | b) |  | A power system consists of two 100 MW units, input cost are given by C1=0.05P12+20P1+800 Rs/Hr and C2=0.06P22+15P2+1000 Rs/Hr, if the total power received is 150MW. - What would be division of load between two units for most economic operation? - Find the savings per hour from economic operation as compared to equal sharing of load of 150 MW. | CO3 | (10) |
| 6. | a) |  | Deduce the condition for optimal load dispatch considering transmission losses in a system. | CO3 | (08) |
|  | b) |  | The figure (6b) shows a system having two plants 1 and 2 connected to buses 1 and 2 respectively. There are two loads and a network of four branches. The reference bus with a voltage of 1.0∠0° PU is shown on the diagram. The branch currents and impedances are Ia=2.0-J0.50 PU Za=0.015+j0.06 PU Ib=1.6-J0.40 PU Zb=0.015+j0.06 PU Ic=1.0-J0.25 PU Zc=0.010+j0.04 PU Id=3.6-J0.90 PU Zd=0.010+j0.04 PU Calculate the loss formula coefficients of the system in PU. | CO3 | (12) |
|  |  |  | UNIT – IV |  |  |
| 7. | a) |  | Explain the swing equation and its importance in stability studies. | CO4 | (05) |
|  | b) |  | What is critical clearing angle and time, explain their role in stability analysis. | CO4 | (05) |
|  | c) |  | With the help of a flow chart, explain the method of finding the transient stability of a given power system using modified Euler’s method. | CO4 | (10) |
| 8. | a) |  | Discuss with relevant figures, the point by point method of solution of swing equation. | CO4 | (10) |
|  | b) |  | Explain the computational algorithm for obtaining the swing curves using Runge Kutta method. | CO4 | (10) |
|  |  |  | UNIT – V |  |  |
| 9. |  | a) | Write a short note on shunt compensator. | CO5 | (05) |
|  |  | b) | Two generators rated 200 MW and 400 MW are operating in parallel. The drop characteristics of their governors are 4% and 5% respectively from no-load to full load. Assuming that the generators are operating at 50Hz at no load. How would a load of 600 MW be shared between them? What will be the system frequency at this load? Assume free governor operation. | CO5 | (05) |
|  |  | c) | Develop a mathematical model for generator-load model and draw the corresponding block diagram. | CO5 | (10) |
| 10. |  | a) | Write a brief note on FACTS controller. | CO5 | (05) |
|  |  | b) | A 100 MVA synchronous generator operates on full load at frequency of 50 Hz. The load is suddenly reduced to 50 MW. Due to time lag in governor system, the steam valve beings to close after 0.4 seconds. Determine the change in frequency that occurs in this time. H=5kW-sec/kVA of generator capacity. | CO5 | (05) |
|  |  | c) | Obtain the expression for change in steady state frequency deviation caused by changes in load demand. | CO5 | (10) |

****************