RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Switchgear and Protection | Max. Marks | : | 100 |
| Subject Code | : | EE71/EE701 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Derive an expression for Restriking Voltage and Rate of Rise of Restriking Voltage (RRRV). | CO1 | (10) |
|  | b) | A circuit breaker interrupts the magnetizing current of a 100 MVA transformer at 220 kV. The magnetizing current of the transformer is 5% of the full load current. Determine the maximum voltage which may appear across the gap of the breaker when the magnetizing current is interrupted at 53% of its peak value. The stray capacitance of 2500 µF and the inductance is 30 H. | CO1 | (05) |
|  | c) | Draw the single line diagram to connect a circuit breaker, Isolator, Earthing switch and write the sequence of operation while opening and closing. | CO1 | (05) |
| 2. | a) | A 3 phase alternator has line to line voltage of 11 kv. The generator is connected to a circuit breaker. The inductive reactance up to the circuit breaker is 5Ω/ph. The distributed capacitance up to the circuit breaker between phase and neutral is 0.01µF. Determine the following: i) Peak restriking voltage across the circuit breaker ii) Frequency of restriking voltage transient iii) Average rate of restriking voltage upto peak restriking voltage iv) Maximum RRRV. | CO1 | (08) |
|  | b) | What are the factors considered in selecting a fuse for: - Transformer protection (ii) Motor protection (iii) Capacitor protection (iv) heaters (v) Lighting loads. | CO1 | (10) |
|  | c) | What are the purposes of using quartz sand as filler material in the construction of HRC fuse? | CO1 | (02) |
|  |  | UNIT- II |  |  |
| 3. | a) | With a neat sketch explain the construction and working of vacuum circuit breaker. | CO2 | (08) |
|  | b) | A circuit breaker is rated as 2000 A, 1500 MVA, 33 kV, 3 phase, 3 seconds, O.C.B. Determine: (i) Rated normal current (ii) rated symmetrical breaking current (iii) Rated making current (iv) short time rating. | CO2 | (06) |
|  | c) | Explain briefly various methods of providing backup protection. | CO2 | (06) |
| 4. | a) | State and explain briefly the characteristics of good protective relaying. | CO2 | (08) |
|  | b) | State the advantages and disadvantages of SF6 circuit breaker. | CO2 | (06) |
|  | c) | Explain the following briefly: (i) Breaking capacity (ii) Short time rating. | CO2 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | With the help of vector diagrams, discuss how an amplitude comparator can be converted to phase comparator and vice versa. | CO3 | (05) |
|  | b) | What is an impedance relay? Discuss how an impedance relay can be realized using i) amplitude comparator ii) phase comparator. Explain its characteristics on the R-X diagram. Discuss the range setting on the three impedance relay placed at a particular location. Discuss why the I zone unit is not set for the protection of 100% of the line. | CO3 | (10) |
|  | c) | The current rating of the relay is 5A, PSM = 1.5, TMS = 1, CT ratio = 400/5, fault current = 6000 A. Determine the operating time of the relay. AS TMS = 1, operating time at various PSM are: PSM; 2; 4; 5; 8; 10; 20. Operating time in secs; 10; 5; 4; 3; 2.8; 2.4. | CO3 | (05) |
| 6. | a) | Fig. 6(a) shows distance protection for a section of a power system. The I zone setting at A and B is 150 ohms (i) what will be impedance seen by the relay at A for a fault at F1? Will the relay at A operate before the circuit breaker at B has tripped? (ii) will the relay at B trip for a fault at F1 before the circuit breaker at A has tripped? (iii) If the circuit breaker C2 fails for a fault at F2, will the fault be cleared by relays at A and B? (iv) How will the fault at F2 be cleared? Fig. 6(a) | CO3 | (10) |
|  | b) | Draw time – current characteristics for different over current relays and compare them based on their applications. | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | What is carrier aided distance protection? What are its different types? Discuss the direct transfer tripping under-reach scheme. | CO4 | (10) |
|  | b) | Explain single phasing in induction motor. How motor is protected from single phasing? | CO4 | (10) |
| 8. | a) | What type of protective scheme employed for the protection of a large power transformer against short circuits? With neat sketches discuss its working principle. | CO4 | (10) |
|  | b) | Describe with a neat sketch, the percentage differential protection of a modern alternator. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | What is adaptive relaying? Discuss protection of transformer using adaptive percentage differential relay. | CO5 | (07) |
|  | b) | With a block diagram, explain numerical over current relay. | CO5 | (08) |
|  | c) | Discuss the applications of PMU in power system. | CO5 | (05) |
| 10. | a) | Discuss WAMS architecture with hierarchy of PMUs and PDCs. | CO5 | (08) |
|  | b) | Describe WAMS based supervision of over-reaching zones of distance relays. | CO5 | (08) |
|  | c) | Write the block diagram of generalised interface for numerical distance relay. | CO5 | (04) |

****************