RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering. | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Switchgear and Protection | Max. Marks | : | 100 |
| Subject Code | : | EE701 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain: i. Resistance Switching ii. Current chopping. | CO1 | (08) |
|  | b) | Explain the selection of fuse for protection of: i) Transformer ii) Motor. | CO1 | (04) |
|  | c) | For a 220 kV system, the reactance and capacitance upto the location of the circuit breaker is 8 ohms and 0.025µF respectively. A resistance of 600 ohms is connected across the contacts of the circuit breaker. Calculate the following: i) Natural frequency of oscillation. ii) Damped frequency of oscillation. iii) Critical value of resistance which will give no transient oscillation. iv) The value of resistance which will give damped frequency of oscillation, one-fourth of the natural frequency of oscillation. | CO1 | (08) |
| 2. | a) | With neat diagram, explain the construction and working of HRC fuse. | CO1 | (10) |
|  | b) | Discuss the recovery rate theory and energy balance theory of arc interruption in a circuit breaker. | CO1 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Discuss the advantages and disadvantages of SF6 circuit breaker. | CO2 | (08) |
|  | b) | With neat diagram, discuss the construction and operating mechanism of vacuum circuit breaker. | CO2 | (12) |
| 4. | a) | With a neat sketch, describe the working principle of an axial air blast type circuit breaker. Explain why resistance switching is used with this type of circuit breaker. | CO2 | (08) |
|  | b) | List the major duties a circuit breaker has to perform under short circuit conditions. Enumerate various types of ratings of a circuit breaker. | CO2 | (12) |
|  |  | UNIT- III |  |  |
| 5. | a) | With a neat sketch explain the working of an Induction cup type relay. | CO3 | (07) |
|  | b) | With a neat circuit diagram explain the working principle of rectifier bridge type amplitude comparator. | CO3 | (05) |
|  | c) | What are the essential qualities of a protective relaying? Explain each one of them. | CO3 | (08) |
| 6. | a) | What are the different types of electromagnetic relays? With a neat sketch explain the construction and working principle of attracted armature type electromagnetic relay. | CO3 | (10) |
|  | b) | List the merits and demerits of static relays over electromagnetic relays. | CO3 | (06) |
|  | c) | With a neat sketch explain the working of phase splitting type amplitude comparator. | CO3 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain with a neat diagram, the principle of distance relaying. | CO4 | (08) |
|  | b) | Explain distance protection by impedance relays. | CO4 | (08) |
|  | c) | List the relays and systems used for transformer protection. | CO4 | (04) |
| 8. | a) | Write a brief note on characteristics of overcurrent relays. | CO4 | (06) |
|  | b) | Explain with the help of neat diagram, the construction and working of non-directional induction type overcurrent relay. | CO4 | (10) |
|  | c) | The current rating of a relay is 5A. PSM = 1.5, CT ratio 400/5, fault current = 6000 A. Determine the operating time of the relay for a TMS = 0.4. At TMS = 1, the operating time at various PSM are, PSM; 2; 4; 5; 8; 10; 20. Operating time (s); 10; 5; 4; 3; 2.8; 2.4. | CO4 | (04) |
|  |  | UNIT- V |  |  |
| 9. | a) | What type of protective scheme employed for the protection of a large power transformer against short circuits? With neat sketches discuss its working principle. | CO5 | (10) |
|  | b) | An 11kV, 100MVA alternator is provided with differential protection. The percentage of winding to be protected against phase to ground fault is 85%. The relay is set to operate when there is 20% out of balance current. Determine the value of the resistance to be placed in the neutral to ground connection. | CO5 | (05) |
|  | c) | Explain protection of rotor against loss of excitation. | CO5 | (05) |
| 10. | a) | What is carrier aided distance protection? What are its different types? Discuss the direct transfer tripping under-reach scheme. | CO5 | (10) |
|  | b) | Explain the advantages of numerical relays. | CO5 | (10) |

****************