RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Switchgear and Protection | Max. Marks | : | 100 |
| Course Code | : | EE71 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Briefly explain the points to be considered while selecting a fuse and bring out the application of the fuse. | CO1 | (10) |
|  | b) | Explain system voltage, restriking voltage, recovery voltage, arc voltage and fault current with the help of neat waveforms. | CO1 | (10) |
| 2. | a) | A 3 phase alternator has line to line voltage of 11 kV. The generator is connected to a circuit breaker. The inductive reactance upto the circuit breaker is 5Ω/ph. The distributed capacitance upto circuit breaker between phase and neutral is 0.01µF. Determine the following - Peak restriking voltage across the circuit breaker - Frequency of restriking voltage transients - Average rate of restriking voltage upto peak restriking voltage - Maximum RRRV. | CO1 | (10) |
|  | b) | With the help of a neat sketch, explain the construction and working principle of a HRC fuse with a tripping device and mention its advantages. | CO1 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | A 33 kV, 250 MVA circuit breaker suddenly closes onto a fault. Determine: i) Symmetrical breaking current ii) Asymmetrical breaking current assuming 75% DC component iii) The peak making current iv) short time current rating. | CO2 | (08) |
|  | b) | with arc quenching medium as the criterion, list different types of circuit breaker and mention their voltage range and breaking capacity. | CO2 | (06) |
|  | c) | Briefly discuss the following terms with respect to protection relay: i) Pickup Value and Drop off value ii) Unit and non unit system of protection iii) Under reach and over reach. | CO3 | (06) |
| 4. | a) | i) Explain CT burden and how is it specified? ii) Explain the basic difference between a CT used for instrumentation and a CT used for protection. | CO2 | (08) |
|  | b) | What is the function of an explosion pot in an oil circuit breaker? Also list different types of explosion pot used in an oil circuit breaker. With suitable sketches, explain their construction, working principle and applications. | CO2 | (12) |
|  |  | UNIT- III |  |  |
| 5. | a) | With the help of a phasor diagrams, explain how amplitude comparator can be converted to phase comparator and vice versa. | CO3 | (10) |
|  | b) | With relevant phasor diagrams, discuss how microprocessor based reactance relay can be realized. | CO3 | (06) |
|  | c) | Under what circumstances are overcurrent relays having very inverse and extremely inverse characteristics used? Briefly explain. | CO3 | (04) |
| 6. | a) | With a suitable sketch, explain how a combination of directional and non-directional relay can be used to protect a simple ring feeder with one infeed and three outfeed. Hence demonstrate time graded protection. | CO3 | (10) |
|  | b) | With a neat block diagram, explain static inverse time over current relay. | CO3 | (06) |
|  | c) | What are the limitations of directional relay? What modification is required to over come the limitation? | CO3 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | A 66 kV, 10 MVA star connected alternator has a reactance of 2 ohms per phase and negligible resistance. Merz-price protection is used for protection of winding. The neutral grounding resistance is 5 ohms. If only 10% of the winding is to remain unprotected, determine the setting of the relay. | CO4 | (08) |
|  | b) | With a neat sketch, explain how a low voltage small sized three phase induction motor is protected for over load, short circuit and under voltage conditions. | CO4 | (08) |
|  | c) | “Plug settings of the differential relay used for protecting a transformer is usually kept higher than that of an alternator”. Give reasons. | CO4 | (04) |
| 8. | a) | With relevant diagram discuss stator overheating protection of an alternator. | CO4 | (07) |
|  | b) | What type of protective scheme is employed for the protection of a large power transformer against internal short circuits? With a neat sketch discuss its working principle. | CO4 | (08) |
|  | c) | With a neat sketch explain the principle of circulating current wire pilot protection. | CO4 | (05) |
|  |  | UNIT- V |  |  |
| 9. | a) | What do you mean by adoptive relaying? With the help of a simple transmission line protection circuit, explain how adoptive relaying helps in optimization of tap setting and leads to elimination of errors in relay operations between different zones. | CO5 | (10) |
|  | b) | Draw the generalized block diagram of microprocessor based distance relay and explain. | CO5 | (10) |
| 10. | a) | With relevant sketch, explain the concept of adaptive control of dependability and security of protection systems based on WAMS. | CO5 | (10) |
|  | b) | With suitable diagram, explain WAM based intelligent load shedding. | CO5 | (10) |

****************