RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS  JANUARY – FEBRUARY 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Switchgear and Protection | Max. Marks | : | 100 |
| Course Code | : | EE71 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | A 3 phase alternator has line to line voltage of 11 kV. The generator is connected to a circuit breaker. The inductive reactance upto the circuit breaker is 5 ohms per phase. The distributed capacitance upto circuit breaker between phase and neutral is 0.01F. Determine the following: i) Peak restriking voltage across the circuit breaker ii) Frequency of restriking voltage transients iii) Average rate of restriking voltage upto peak restriking voltage iv) Maximum RRRV. | CO1 | (08) |
|  | b) | With a neat sketch, explain the working of HRC fuse. Bring out the advantages and disadvantages of HRC fuse. | CO1 | (07) |
|  | c) | Explain Energy Balance theory and Recovery rate theory. | CO1 | (05) |
| 2. | a) | Derive an expression for Restriking Voltage and Rate of Rise of Restriking Voltage (RRRV). | CO1 | (08) |
|  | b) | Distinguish between fuse and a circuit breaker. Also describe the essential requirements of a fuse material. | CO1 | (08) |
|  | c) | Define the following terms with respect to fuse: i) Fusing factor ii) Breaking capacity iii) Prospective current iv) Minimum fusing current. | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | What do you understand by zones of protection? With a neat single line diagram discuss various zones of protection for a modern power system and hence bring out the concept of backup protection. | CO3 | (10) |
|  | b) | What are the fundamental requirement of protective relaying? Briefly explain each one of them. | CO3 | (05) |
|  | c) | A three phase circuit is rated at 1250 A, 2000 MVA, 33kV, 4 seconds. Find the rated symmetrical breaking current, making current and short time rating. | CO2 | (05) |
| 4. | a) | An 11 kV, 500 MVA circuit breaker suddenly closes on to a fault. Determine: i) the symmetrical breaking current ii) the asymmetrical breaking current assuming 50% DC component iii) the peak making current iv) the short time rating. | CO2 | (07) |
|  | b) | With a neat sketch, explain the construction and working of a double pressure SF6 circuit breaker. Bring out the merits of SF6 circuit breaker. | CO2 | (08) |
|  | c) | Bring out the classification of protective relay based on i) Technology ii) Functionality. Briefly explain. | CO3 | (05) |
|  |  | UNIT – III |  |  |
| 5. | a) | What are the advantages of static relay over electromagnetic relay. | CO3 | (05) |
|  | b) | Discuss how an amplitude comparator can be converted to phase comparator and vice versa. | CO3 | (10) |
|  | c) | Explain what are amplitude and phase comparators. | CO3 | (05) |
| 6. | a) | With neat diagram explain the working principle of MHO relay. | CO3 | (05) |
|  | b) | With neat diagram explain the principle of induction type directional over current relay. | CO3 | (10) |
|  | c) | With neat diagram explain the working principle of impedance relay. | CO3 | (05) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Which are the various types of faults which can occur in a generator. | CO4 | (05) |
|  | b) | Explain the Merz-Price Protection of alternator stator windings. | CO4 | (10) |
|  | c) | Explain the frame leakage protection. | CO4 | (05) |
| 8. | a) | Explain the abnormal conditions and possible failure of Induction motors. | CO4 | (05) |
|  | b) | With a neat diagram, explain the working of protection provided for 3-Phase induction motor against “single phasing”. | CO4 | (10) |
|  | c) | With a neat diagram, Explain the working of Buchholz relay. | CO4 | (05) |
|  |  | UNIT – V |  |  |
| 9. | a) | Explain the following: i) Phasor Measurement Unit ii) Phasor Data Concentrator iii) Wide Area Measuring Systems. | CO5 | (08) |
|  | b) | What do you mean by adoptive relaying? With the help of a simple transmission line protection circuit, explain how adoptive relaying helps in optimization of tap setting and leads to elimination of errors in relay operations between different zones. | CO5 | (12) |
| 10. | a) | With relevant diagram, explain the following: i) supervision of backup zones ii) load shedding and restoration. | CO5 | (10) |
|  | b) | Draw the block diagram of microprocessor based over current relay and explain. | CO5 | (10) |

****************