RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY/AUGUST 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Switchgear and Protection | Max. Marks | : | 100 |
| Subject Code | : | EE701 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | State and derive the fuse law for a circular fuse element. | CO1 | (08) |
|  | b) | Explain the following: - Initiation of arc ii) Maintenance of arc iii) Arc voltage. | CO1 | (06) |
|  | c) | An 11kV, 500MVA, circuit breaker suddenly closes on to a fault. Determine i) the symmetrical breaking current ii) the asymmetrical breaking current assuming 50% dc component iii) the peak making current and iv) the short-time current rating of the circuit breaker. | CO1 | (06) |
| 2. | a) | A 50Hz, 11kV, three phase alternator with earthed neutral has a resistance of 5Ω /phase and is connected to bus bar through a circuit breaker. The capacitance to earth between the alternator and the circuit breaker is 0.02μF /phase. Assuming the resistance of the generator to be negligible. calculate the following: i) Maximum voltage across the contacts of the circuit breaker. ii)Frequency of oscillations. iii) The average rate of rise of restriking voltage up to the first peak. | CO1 | (10) |
|  | b) | Discuss the phenomenon of current chopping in a circuit breaker. | CO1 | (06) |
|  | c) | A current of 6A fuses a lead fuse of diameter 0.0195 inch. What current will fuse a wire of 0.0392 inch? | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | With neat diagram, describe axial blast pot type oil circuit breaker. | CO2 | (06) |
|  | b) | Explain with neat sketche Puffer type SF6 circuit breaker. | CO2 | (08) |
|  | c) | Explain advantages and disadvantages of air blast circuit breakers over oil circuit breakers. | CO2 | (06) |
| 4. | a) | What are the various types of operating mechanism which are used for opening and closing of the contacts of a circuit breaker? Discuss their merits and demerits. | CO2 | (10) |
|  | b) | What are the different types of air blast circuit breaker? Discuss their operating principle. Which type is less prone to current chopping? | CO2 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Describe the essential qualities of protection. | CO3 | (10) |
|  | b) | Discuss how an amplitude comparator can be converted to phase comparator, and vice versa. | CO3 | (10) |
| 6. | a) | What do you understand by zones of protection? Discuss various zones of protection for modern power system. | CO3 | (08) |
|  | b) | Explain primary and back-up protection. What are the various methods of providing back-up protection? | CO3 | (12) |
|  |  | UNIT- IV |  |  |
| 7. |  | What is an impedance relay? Explain its operating principle. Discuss how it is realised using the i) electromagnetic principle ii) amplitude comparator iii) phase comparator. | CO4 | (20) |
| 8. | a) | Describe the constructional features and operating principle of directional overcurrent relay. | CO4 | (10) |
|  | b) | With a neat sketch, explain the operation of Buchholz relay. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | What are the consequences of loss of excitation of a large generator? Explain how the generator can be protected | CO5 | (10) |
|  | b) | A 11 kV, 100 MVA alternator is grounded through a resistance of 5Ω. The CTs have a ratio of 1000/5. The relay is set to operate where there is an out of balance current of 1A. What percentage of the generator winding will be protected by the percentage differential scheme of protection? | CO5 | (06) |
|  | c) | List the different stator and rotor protection schemes for the protection of generators. | CO5 | (04) |
| 10. | a) | A star-connected, 3 phase, 10MVA, 6.6kV alternator has a per phase reactance of 10%. It is protected by Merz-Price circulating-current principle, which is set to operate for fault currents not less than 175A. Calculate the value of earthing resistance to be provided in order to ensure that only 10% of the alternator winding remains unprotected. | CO5 | (10) |
|  | b) | Describe the Merz-Price circulating current system for the protection of transformers. | CO5 | (10) |

****************