RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  JULY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Switchgear and Protection | Max. Marks | : | 100 |
| Subject Code | : | EE71 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the points to be considered into account while selecting a fuse. | CO1 | (06) |
|  | b) | Explain: i. Resistance Switching ii. Current chopping, | CO1 | (08) |
|  | c) | For a 132kV system, the reactance and capacitance up to the location of the circuit breaker is 3Ω and 0.015µF, respectively. Calculate: i) The frequency of transient oscillation. ii) The maximum value of restriking voltage across the contacts of the circuit breaker. iii) The maximum value of RRRV. | CO1 | (06) |
| 2. | a) | With neat diagram, explain the construction and working of HRC fuse. | CO1 | (10) |
|  | b) | Discuss the recovery rate theory and energy balance theory of arc interruption in a circuit breaker. | CO1 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | What are the different types of circuit breaker when the arc quenching medium is the criterion? Mention the voltage range for which a particular type of circuit breaker is recommended. | CO2 | (06) |
|  | b) | Explain Puffer type SF6 circuit breaker. | CO2 | (06) |
|  | c) | Describe the essential qualities of protection. | CO2 | (08) |
| 4. | a) | What are the various types of operating mechanism which are used for opening and closing of the contacts of a circuit breaker? Discuss their merits and demerits. | CO2 | (08) |
|  | b) | What are the different types of air blast circuit breaker? Discuss their operating principle. Which type is less affected by current chopping? | CO2 | (12) |
|  |  | UNIT- III |  |  |
| 5. | a) | Discuss how an amplitude comparator can be converted to phase comparator, and vice versa. | CO3 | (08) |
|  | b) | With a block diagram, discuss definite time overcurrent relay. | CO3 | (06) |
|  | c) | Describe induction cup type impedance relay. | CO3 | (06) |
| 6. | a) | Discuss the operating principle of a rectifier bridge phase comparator. | CO3 | (06) |
|  | b) | With neat diagram, explain construction and working of non-directional Induction type overcurrent relay. | CO3 | (08) |
|  | c) | What is impedance relay? Explain its operating principle. | CO3 | (06) |
|  |  | UNIT- IV |  |  |
| 7. | a) | The neutral point of a 10kV alternator is earthed through a resistance of 10 Ω. The relay is set to operate when there is an out of balance current of 1 A. the CTs have a ratio of 1000/5. What percentage of the winding is protected against fault to earth and what must be the minimum value of earthing resistance to give 90 % protection to each phase winding. | CO4 | (08) |
|  | b) | Explain the protection of generator against loss of excitation. | CO4 | (06) |
|  | c) | With a neat diagram explain the working of a buchholz relay. | CO4 | (06) |
| 8. | a) | Describe the principle of Merz Price protection of a star-star connected transformer. | CO4 | (08) |
|  | b) | Explain how an induction motor is protected against phase failure. | CO4 | (06) |
|  | c) | Write a brief note on carrier aided distance protection of transmission lines. | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | With a neat block diagram, explain the realization of directional over current relay using a microprocessor. | CO5 | (10) |
|  | b) | With a neat sketch explain the architecture of WAMS for power systems. | CO5 | (10) |
| 10. | a) | Briefly discuss the concept of adaptive relaying. | CO5 | (05) |
|  | b) | With a neat block diagram, explain the realization of directional impedance relay using a microprocessor. | CO5 | (10) |
|  | c) | With a neat sketch explain the placement of PMUs in a power system. | CO5 | (05) |

****************